- Huawei - MBTS Swap Completion Report

(To be completed by site supervisor after hardware installation)

| Company Name | RADCOM | Site Supervisor | MIHALACHE George |  |
| --- | --- | --- | --- | --- |
| Huawei Supervisor |  |  | Tel. | 0749 110 073 |
| Site Address | Nicolae Titulescu | Site Type | GreenField |  |
| Site name | 5606 | MBTS name | GreenField |  |
| Stop server time | 1.0 | Start server time | 3.30 |  |

|  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | No |  | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hardware Check-up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Insure DIP switches of master and slave BBU, APM PSM are set correctly. (DBS3900 in APM) |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 2 | All of RF connectors in RRU side are tight. |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 3 | Inner wires are properly connected. |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 4 | All of grounding cables are connected correctly. |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 5 | Jumper labels must be properly stuck. |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| 6.1 |  | 3G Site Verification (Adjust the mobile in debug model collect the related information of each cell around the site) Perform voice call 5 times, VP 5 times, PS384K service 2 times in all sectors. Record the number of success |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ITEMS |  |  | Cell id |  | P-SC | RSCP |  | CPICH Ec/Io |  | Voice al | VP |  |  |  | PS384K |  |
| Sector 1 |  |  | 56061 |  | 119 |  |  |  |  | X |  |  |  |  | X |  |
| Sector 2 |  |  | 56062 |  | 286 |  |  |  |  | X |  |  |  |  | X |  |
| Sector 3 |  |  | 56063 |  | 453 |  |  |  |  | x |  |  |  |  | X |  |
| Sector 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note: from the above form, can prevent the TX cross-feeders on each sector. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.2 |  | Softer handover testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Make a voice call in front of one sector and drive around the tested cell site, to a distance between 50 and 300 meters from the site. The site will be rounded twice, one in each moving sense. The call should be maintained. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Handover |  |  |  | P-SC |  |  | Passed |  | Failed |  |  |  | Comments |  |  |  |
| Sector 1 to 2 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| Sector 2 to 3 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| Sector 3 to 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sector 4 to 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sector 1 to 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sector 4 to 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sector 3 to 2 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| Sector 2 to 1 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| 7 |  | 2G Site Verification (Adjust the mobile in debug model collect the related information of each cell around the site) Perform voice call 5 times, Packet service 2 times in all sectors. Record the number of success. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ITEMS |  |  |  | Cell id |  |  | Frequency(BCCH) |  | RxLev / RxQual |  |  |  | Packet Service |  |  |  |
| Sector 1 |  |  |  | 56061 |  |  | 26 |  |  |  |  |  |  |  |  |  |
| Sector 2 |  |  |  | 56062 |  |  | 59 |  |  |  |  |  |  |  |  |  |
| Sector 3 |  |  |  | 56063 |  |  | 20 |  |  |  |  |  |  |  |  |  |
| Sector 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sector 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sector 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sector 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remark |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Subcontractor measurement technician have to walk approximate 200 m away from site in each sector direction! The OMC engineer / Huawei should start the IMSI trace in 3G and 2G network before the integration test. The files will be posted on REMEDY – Site Integration SRNM (in the same folder with the integration acceptance form) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 |  | 2G handover testing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Make a voice call in front of one sector and drive around the tested cell site, to a distance between 50 and 300 meters from the site. The site will be rounded twice, one in each moving sense. The call should be maintained. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Handover |  |  |  | Cell id |  |  | Frequency(BCCH) from initial to final neighbour cell |  | RxLev / RxQual |  |  |  | Packet Service |  |  |  |
| Sector 1 to 2 |  |  |  |  |  |  | 26/59 |  |  |  |  |  |  |  |  |  |
| Sector 2 to 3 |  |  |  |  |  |  | 59/20 |  |  |  |  |  |  |  |  |  |
| Sector 3 to 4 |  |  |  |  |  |  | / |  |  |  |  |  |  |  |  |  |
| Sector 4 to 1 |  |  |  |  |  |  | / |  |  |  |  |  |  |  |  |  |
| Sector 1 to 4 |  |  |  |  |  |  | / |  |  |  |  |  |  |  |  |  |
| Sector 4 to 3 |  |  |  |  |  |  | / |  |  |  |  |  |  |  |  |  |
| Sector 3 to 2 |  |  |  |  |  |  | 20/59 |  |  |  |  |  |  |  |  |  |
| Sector 2 to 1 |  |  |  |  |  |  | 59/26 |  |  |  |  |  |  |  |  |  |

Comments

site supervisor

Signature MIHALACHE George