# RRU5909s Technical Specifications

## Contents

6.1.1.6.6.1 RRU5909s Technical Specifications

# 6.1.1.6.6.1 RRU5909s Technical Specifications

An RRU5909s is a multimode radio remote unit. Adopting the software-defined radio (SDR) technology, RRU5909s modules can work in different modes through software configurations.

#### Supported Modes and Frequency Bands

The following table lists the modes and frequency bands supported by an RRU5909s.

| Table 1 Modes and frequency bands supported by an RRU5909s |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Frequency Band (MHz) | Receive Frequency Band (MHz) | Transmit Frequency Band (MHz) | Mode | IBW (MHz) |
| RRU5909s | 1800 | 1710 to 1785 | 1805 to 1880 | LTE FDD and GL | 45 |
|  | 2100 | 1920 to 1980 | 2110 to 2170 | UMTS, LTE FDD, and UL | 60 |

#### RF Specifications

The following table lists radio frequency (RF) specifications of an RRU5909s.

NOTE:

- ATBR in the Transmit and Receive Channels column indicates that this RF module has A transmit channels and B receive channels.

- CxD W in the Output Power column indicates that this RF module is configured with C transmit channels and the maximum output power of each transmit channel is D W.

- EF MSR indicates that E and F data is carried on the same transmit channel of an RF module.

- EF non-MSR indicates that E and F data is carried on two different transmit channels of an RF module.

- GSM receiver sensitivity is measured, as recommended in 3GPP TS 51.021, in the central band (80% of the full band) at the antenna connector on condition that the channel rate reaches 13 kbit/s and the bit error rate (BER) is not higher than 2%.

- UMTS receiver sensitivity is measured, as recommended in 3GPP TS 25.104, at the antenna connector on condition that the channel rate reaches 12.2 kbit/s and the BER is not higher than 0.001.

- LTE FDD receiver sensitivity is measured, as recommended in 3GPP TS 36.104, under a 5 MHz channel bandwidth based on the FRC A1-3 in Annex A.1 (QPSK, R = 1/3, 25 RBs) standard.

- The RRU5909s that works in LTE FDD or multi-standard radio (MSR) mode and operates in the 1800 MHz frequency band complies with the ETSI EN 301 908 V5.2.1 and 3GPP TS 37.104 standards.

- RRU5909s modules that work in UMTS, LTE FDD, or MSR mode and operate in the 2100 MHz frequency band comply with the 3GPP TS 37.104 V10.4.0 and TS 37.141 V10.4.0 standards.

| Table 2 RF specifications of an RRU5909s |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Transmit and Receive Channels | Capacity | Receiver Sensitivity (dBm) |  |  | Output Power | Power Consumption |
|  |  |  | Receiver Sensitivity with One Antenna | Receiver Sensitivity with Two Antennas | Receiver Sensitivity with Four Antennas |  |  |
| RRU5909s | 2T2R | GSM: 6 TRXs UMTS: - Without MIMO: 8 carriers - With MIMO: 4 carriers LTE FDD: - 1800 MHz: 2 carriers. The bandwidth per carrier is 1.4, 3, 5, 10, 15, or 20 MHz. - 2100 MHz: 2 carriers. The bandwidth per carrier is 5, 10, 15, or 20 MHz. | GSM: -114.0 UMTS: -126.1 LTE FDD: -107.0 | GSM: -116.8 UMTS: -128.9 LTE FDD: -109.8 | GSM: -119.5 (theoretical value) UMTS: -131.6 LTE FDD: -112.5 | The RRU5909s supports the maximum power configuration of 2 x 60 W. For typical configurations, see Typical Power Configuration for RRU5909s. | See the RRU5909s sheet in 3900 Series Base Station Power Consumption Specifications. |

#### Engineering Specifications

The following table lists the equipment specifications of an RRU5909s.

| Table 3 Equipment specifications of an RRU5909s |  |  |  |
| --- | --- | --- | --- |
| Type | Input Power | Dimension (H x W x D) | Weight (kg) |
| RRU5909s | -48 V DC; voltage range: -36 V DC to -57 V DC NOTE: The RRU5909s supports AC power supply when connected to an external AC/DC power module or an OPM15M. For details, see AC/DC Power Module User Guide and OPM15M User Guide. | 400 mm x 300 mm x 70 mm (without the housing) | 12 (without the housing) |

The following table lists the environmental specifications of an RRU5909s.

| Table 4 Environmental specifications of an RRU5909s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Operating Temperature | Relative Humidity | Absolute Humidity | Atmospheric Pressure |
| RRU5909s | - With solar radiation: -40°C to +50°C - Without solar radiation: -40°C to +55°C NOTE: According to the installation scenario, traffic load, and carrier configuration, the output power of some RRUs may temporarily decrease when they work within the range of 10°C higher than the allowed highest temperature. | 5% RH to 100% RH | 1 g/m3 to 30 g/m3 | 70 kPa to 106 kPa |

The following table lists the compliance standards for an RRU5909s.

| Table 5 Compliance standards for an RRU5909s |  |
| --- | --- |
| Item | Standard |
| RF | - 3GPP TS 45.005 - 3GPP TS 25.141 - 3GPP TS 36.141 - 3GPP TS 37.141 |
| Operating environment | ETSI EN 300019-1-4 V2.1.2 (2003-04) Class 4.1: "Non-weatherprotected locations" |
| Storage environment | ETSI EN300019-1-1 V2.1.4 (2003-04) class1.2 "Weather protected, not temperature-controlled storage locations" NOTE: - The validity period is one year. - The product can function properly within the validity period if the storage environment meets the preceding standards. |
| Shockproof protection | NEBS GR63 zone4 |
| Ingress protection (IP) rating | IP65 |
| Surge protection | - IEC 62305-1 Protection against lightning - Part 1:General principles - IEC 62305-3 Protection against lightning - Part 3:Physical damage to structures and life hazard - IEC 62305-4 Protection against lightning - Part 4:Electrical and electronic systems within structures - ITU-T K.35 Bonding configurations and earthing at remote electronic sites - ITU-T K.56 Protection of radio base stations against lightning discharges - ITU-T K.97 Lightning protection of distributed base stations - ETSI EN 300 253 Environmental Engineering(EE):Earthing and bonding configuration inside telecommunications centers - YD/T 2324-2011 Lightning protection requirements and test methods for Radio Base Stations (RBSs) - GB 50689-2011 Code for design of lightning protection and earthing engineering for telecommunication bureaus (stations) |

Table 6 lists the surge protection specifications of ports on an RRU5909s.

NOTE:

- Unless otherwise specified, the surge protection specifications depend on the surge waveform of 8/20 μs.

- All the surge current items, unless otherwise specified as Maximum discharge current, refer to Nominal discharge current.

| Table 6 Surge protection specifications of ports on an RRU5909s |  |  |
| --- | --- | --- |
| Port | Surge Protection Mode | Surge Protection Specifications |
| Power supply port | Surge current | 20 kA |

#### Specifications of CPRI Ports

The following table lists the specifications of CPRI ports on an RRU5909s.

| Table 7 Specifications of CPRI ports on an RRU5909s |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Number of CPRI Ports | CPRI Date Rate (Gbit/s) | Topology | Cascading Level | Maximum Distance from the BBU (km) |
| RRU5909s | 2 | 1.25, 2.5, 4.9, or 9.8 | Star, chain, or dual-star | CPRI MUX: - GL: 4 - UL: 4 | GL/UL: The maximum distance from the BBU varies with the types of LTE BBPs configured. - When the LBBPc, or LBBPd1 is configured: 20 - When the LBBPd2, UBBPd3, UBBPd4, UBBPd5, UBBPd6, or UBBPe is configured: 40 - When the LBBPd3 is configured - 40 if the cell number is less than or equal to 3 - 20 if the cell number is greater than or equal to 4 |

#### Antenna Capability

The following table lists the antenna capability of an RRU5909s.

| Table 8 Antenna capability of an RRU5909s |  |  |
| --- | --- | --- |
| Type | TMA Support | Supported RET Antenna |
| RRU5909s | Supported | AISG2.0 and AISG1.1 |

NOTE:

For RF modules supporting RET antennas, the feeding voltage is 12 V and feeding current is 2.3 A.

Parent Topic: Technical Specifications of RRUs

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

Next topic >