# Typical Power Configuration for RRU3959a

## Contents

6.1.1.6.10.3.19 Typical Power Configuration for RRU3959a

# 6.1.1.6.10.3.19 Typical Power Configuration for RRU3959a

An RRU3959a is a 2TX RF module that can work in GSM, UMTS, LTE FDD, GL, GU, UL, GUL, GM, UM, LM , GLM mode.

- Table 1 and Table 2 provide the typical power configuration for the RRU3959a (GSM, 900 MHz/1800 MHz).

- Table 3 and Table 4 provide the typical power configuration for the RRU3959a (UMTS, 900 MHz/2100 MHz).

- Table 5 and Table 6 provide the typical power configuration for the RRU3959a (LTE FDD, 900 MHz).

- Table 7 and Table 8 provide the typical power configuration for the RRU3959a (LTE FDD, 1800 MHz/2100 MHz).

- Table 9 and Table 10 provide the typical power configuration for the RRU3959a (GU non-MSR, 900 MHz).

- Table 11 and Table 12 provide the typical power configuration for the RRU3959a (GU MSR, 900 MHz).

- Table 13 and Table 14 provide the typical power configuration for the RRU3959a (GL non-MSR, 1800 MHz).

- Table 15 and Table 16 provide the typical power configuration for the RRU3959a (GL MSR, 900 MHz).

- Table 17 and Table 18 provide the typical power configuration for the RRU3959a (GL MSR, 1800 MHz).

- Table 19 and Table 20 provide the typical power configuration for the RRU3959a (UL MSR, 900 MHz).

- Table 21 and Table 22 provide the typical power configuration for the RRU3959a (UL MSR, 2100 MHz).

- Table 23 and Table 24 provide the typical power configuration for the RRU3959a (GUL MSR, 900 MHz).

- Table 25 and Table 26 provide the typical power configuration for the RRU3959a (LTE NB-IoT, 900 MHz/1800 MHz).

- Table 27 and Table 28 provide the typical power configuration for the RRU3959a (GM, 900 MHz/1800 MHz).

- Table 29 and Table 30 provide the typical power configuration for the RRU3959a (UM, 900 MHz).

- Table 31 and Table 32 provide the typical power configuration for the RRU3959a (LM, 900 MHz/1800 MHz).

- Table 33 and Table 34 provide the typical power configuration for the RRU3959a (GLM, 1800 MHz).

NOTE:

- If the power sharing feature is activated, assume that UEs in a cell are randomly located.

- If the RF module is placed at an altitude of 3500 to 4500 meters, its power reduces by 1 dB. If the RF module is placed at an altitude of 4500 to 6000 meters, its power reduces by 2 dB.

- Station spacing, frequency multiplexing factor, power control algorithm, and traffic model all affect the gains of dynamic power sharing. In most cases, network plans are designed on the basis of power specifications of dynamic power sharing.

- Before activating the dynamic power sharing feature, enable the DTX and power control functions. In GBSS8.1, the dynamic power sharing feature is mutually exclusive with the GBFD-113201 Concentric Cell, GBFD-114501 Co-BCCH Cell, GBFD-118001 BCCH Dense Frequency Multiplexing, and GBFD-117501 Enhanced Measurement Report (EMR) features. In GBSS9.0 and later versions, the dynamic power sharing feature can be used together with these features. However, the dynamic power sharing feature currently cannot be used together with the GBFD-117002 IBCA (Interference Based Channel Allocation), GBFD-117001 Flex MAIO, GBFD-118701 RAN Sharing, and GBFD-114001 Extended Cell features in GBSS8.1, GBSS9.0, and later versions.

- For the RRU3959a working in GSM mode, the maximum output power per carrier is 60 W when the S1 or S2 configuration is used. If the output power of 60 W is required, the corresponding power license must be purchased.

- When two LTE carriers are configured, it is recommended that the power spectrum density (PSD) of the two carriers be set to the same value. Power spectrum density = Carrier output power/Carrier bandwidth (1.4 MHz and 3 MHz bandwidths are considered as 5 MHz bandwidth in this formula.)

| Table 1 Output power of the RRU3959a (GSM, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAT |  | Total Number of GSM TRXs |  |  |  | Output Power per GSM TRX (W) |  |  |  | Output Power per GSM TRX with Dynamic Power Sharing (W) |  |
| GSM |  | 1 |  |  |  | 60 |  |  |  | 60 |  |
|  |  | 2 |  |  |  | 60 |  |  |  | 60 |  |
|  |  | 3 |  |  |  | 30 |  |  |  | 30 |  |
|  |  | 4 |  |  |  | 30 |  |  |  | 30 |  |
|  |  | 5 |  |  |  | 20 |  |  |  | 25 |  |
|  |  | 6 |  |  |  | 20 |  |  |  | 25 |  |
|  |  | 7 |  |  |  | 15 |  |  |  | 20 |  |
|  |  | 8 |  |  |  | 15 |  |  |  | 20 |  |
| Table 2 Configuration of RRU3959a PAs (GSM, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of GSM TRXs |  | Output Power per GSM TRX (W) |  | Output Power per GSM TRX with Dynamic Power Sharing (W) |  | Number of GSM TRXs |  | Output Power per GSM TRX (W) |  | Output Power per GSM TRX with Dynamic Power Sharing (W) |
| GSM 1 | 1 |  | 60 |  | 60 |  | 0 |  | 0 |  | 0 |
| GSM 2 | 1 |  | 60 |  | 60 |  | 1 |  | 60 |  | 60 |
| GSM 3 | 2 |  | 30 |  | 30 |  | 1 |  | 30 |  | 30 |
| GSM 4 | 2 |  | 30 |  | 30 |  | 2 |  | 30 |  | 30 |
| GSM 5 | 3 |  | 20 |  | 25 |  | 2 |  | 20 |  | 25 |
| GSM 6 | 3 |  | 20 |  | 25 |  | 3 |  | 20 |  | 25 |
| GSM 7 | 4 |  | 15 |  | 20 |  | 3 |  | 15 |  | 20 |
| GSM 8 | 4 |  | 15 |  | 20 |  | 4 |  | 15 |  | 20 |
| Table 3 Output power of the RRU3959a (UMTS, 900 MHz/2100 MHz) |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  |  |  | Total Number of UMTS Carriers |  |  |  | Output Power per UMTS Carrier (W) |  |  |  |
| UMTS |  |  |  | 1 |  |  |  | 60 |  |  |  |
|  |  |  |  | 2 |  |  |  | 60 |  |  |  |
|  |  |  |  | 3 |  |  |  | 30 |  |  |  |
|  |  |  |  | 4 |  |  |  | 30 |  |  |  |
|  |  |  |  | 5 |  |  |  | 20 |  |  |  |
|  |  |  |  | 6 |  |  |  | 20 |  |  |  |
|  |  |  |  | 1 (MIMO) |  |  |  | 2x60 |  |  |  |
|  |  |  |  | 2 (MIMO) |  |  |  | 2x30 |  |  |  |
|  |  |  |  | 3 (MIMO) |  |  |  | 2x20 |  |  |  |
|  |  |  |  | 4 (MIMO) |  |  |  | 2x15 |  |  |  |
|  |  |  |  | 2 + 1 (MIMO) |  |  |  | Non-MIMO carrier:30 MIMO carrier:2x30 |  |  |  |
|  |  |  |  | 2 + 2 (MIMO) |  |  |  | Non-MIMO carrier:20 MIMO carrier: 2x20 |  |  |  |
|  |  |  |  | 2 + 3 (MIMO) |  |  |  | Non-MIMO carrier:15 MIMO carrier: 2x15 |  |  |  |
|  |  |  |  | 4 + 1 (MIMO) |  |  |  | Non-MIMO carrier:20 MIMO carrier: 2x20 |  |  |  |
|  |  |  |  | 4 + 2 (MIMO) |  |  |  | Non-MIMO carrier:15 MIMO carrier: 2x15 |  |  |  |
|  |  |  |  | 6 + 1 (MIMO) |  |  |  | Non-MIMO carrier:15 MIMO carrier: 2x15 |  |  |  |

| Table 4 Configuration of RRU3959a PAs (UMTS, 900 MHz/2100 MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  | PA2 |  |  |
|  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |
| UMTS 1 | 1 |  | 60 |  | 0 |  | 0 |
| UMTS 2 | 1 |  | 60 |  | 1 |  | 60 |
| UMTS 3 | 2 |  | 30 |  | 1 |  | 30 |
| UMTS 4 | 2 |  | 30 |  | 2 |  | 30 |
| UMTS 5 | 3 |  | 20 |  | 2 |  | 20 |
| UMTS 6 | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 1 (MIMO) | 1 |  | 60 |  | 1 |  | 60 |
| UMTS 2 (MIMO) | 2 |  | 30 |  | 2 |  | 30 |
| UMTS 3 (MIMO) | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 4 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| UMTS 2 + 1 (MIMO) | 2 |  | 30 |  | 2 |  | 30 |
| UMTS 2 + 2 (MIMO) | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 2 + 3 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| UMTS 4 + 1 (MIMO) | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 4 + 2 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| UMTS 6 + 1 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| Table 5 Output power of the RRU3959a (LTE FDD, 900 MHz) |  |  |  |  |  |  |  |
| RAT |  | Total Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| LTE FDD |  | 1 (MIMO) |  | 2x60 |  | 5, 10, 15, 20 |  |
|  |  | 1 (MIMO) |  | 2x40 |  | 1.4, 3 |  |
|  |  | 2 (MIMO) |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 (MIMO) |  | Carrier 1: 2x20 Carrier 2: 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |

| Table 6 Configuration of RRU3959a PAs (LTE FDD, 900 MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  | PA2 |  |
|  | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |
| LTE FDD 1 (MIMO) | 1 | 60 | 1 | 60 |
| LTE FDD 1 (MIMO) | 1 | 40 | 1 | 40 |
| LTE FDD 2 (MIMO) | 2 | 30 | 2 | 30 |
| LTE FDD 2 (MIMO) | 2 | Carrier 1: 20 Carrier 2: 40 | 2 | Carrier 1: 20 Carrier 2: 40 |

NOTE:

In Table 7 and Table 8, (a) indicates that the configuration is supported only when an RRU3959a operates in the 1800 MHz frequency band.

| Table 7 Output power of the RRU3959a (LTE FDD, 1800 MHz/2100 MHz) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAT |  |  | Total Number of LTE FDD Carriers |  |  | Output Power per LTE FDD Carrier (W) |  |  | Bandwidth per LTE FDD Carrier (MHz) |  |  |
| LTE FDD |  |  | 1 (MIMO) |  |  | 2x60 |  |  | 1800 MHz: 1.4, 3, 5, 10, 15, 20 2100 MHz: 5, 10, 15, 20 |  |  |
|  |  |  | 2 (MIMO) |  |  | 2x30 |  |  | 1800 MHz: 1.4, 3, 5, 10, 15, 20 2100 MHz: 5, 10, 15, 20 |  |  |
|  |  |  | 2 (MIMO)(a) |  |  | Carrier 1: 2x20(a) Carrier 2: 2x40(a) |  |  | 1800 MHz: 1.4, 3, 5, 10, 15, 20(a) |  |  |
| Table 8 Configuration of RRU3959a PAs (LTE FDD, 1800 MHz/2100 MHz) |  |  |  |  |  |  |  |  |  |  |  |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of LTE FDD Carriers |  |  | Output Power per LTE FDD Carrier (W) |  |  | Number of LTE FDD Carriers |  |  |  | Output Power per LTE FDD Carrier (W) |
| LTE FDD 1 (MIMO) | 1 |  |  | 60 |  |  | 1 |  |  |  | 60 |
| LTE FDD 2 (MIMO) | 2 |  |  | 30 |  |  | 2 |  |  |  | 30 |
| LTE FDD 2 (MIMO)(a) | 2(a) |  |  | Carrier 1: 20(a) Carrier 2: 40(a) |  |  | 2(a) |  |  |  | Carrier 1: 20(a) Carrier 2: 40(a) |
| Table 9 Output power of the RRU3959a (GU non-MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of GSM TRXs |  |  | Total Number of UMTS Carriers |  |  | Output Power per GSM TRX (W) |  | Output Power per UMTS Carrier (W) |  |
| GSM + UMTS |  | 1 |  |  | 1 |  |  | 60 |  | 60 |  |
|  |  | 2 |  |  | 1 |  |  | 30 |  | 60 |  |
|  |  | 3 |  |  | 1 |  |  | 20 |  | 60 |  |
|  |  | 4 |  |  | 1 |  |  | 15 |  | 60 |  |
|  |  | 5 |  |  | 1 |  |  | 10 |  | 60 |  |
|  |  | 6 |  |  | 1 |  |  | 7 |  | 60 |  |
|  |  | 1 |  |  | 2 |  |  | 60 |  | 30 |  |
|  |  | 2 |  |  | 2 |  |  | 30 |  | 30 |  |
|  |  | 3 |  |  | 2 |  |  | 20 |  | 30 |  |
|  |  | 4 |  |  | 2 |  |  | 15 |  | 30 |  |
|  |  | 5 |  |  | 2 |  |  | 10 |  | 30 |  |
|  |  | 6 |  |  | 2 |  |  | 7 |  | 30 |  |
|  |  | 1 |  |  | 3 |  |  | 60 |  | 20 |  |
|  |  | 2 |  |  | 3 |  |  | 30 |  | 20 |  |
|  |  | 3 |  |  | 3 |  |  | 20 |  | 20 |  |
|  |  | 4 |  |  | 3 |  |  | 15 |  | 20 |  |
|  |  | 5 |  |  | 3 |  |  | 10 |  | 20 |  |
|  |  | 2 |  |  | 4 |  |  | 30 |  | 15 |  |
|  |  | 3 |  |  | 4 |  |  | 20 |  | 15 |  |
|  |  | 4 |  |  | 4 |  |  | 15 |  | 15 |  |

| Table 10 Configuration of RRU3959a PAs (GU non-MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  | PA2 |  |  |  |  |  |
|  | Number of GSM TRXs |  | Output Power per GSM TRX (W) | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) | Number of GSM TRXs |  | Output Power per GSM TRX (W) | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |
| GSM 1 + UMTS 1 | 1 |  | 60 | 0 |  | 0 | 0 |  | 0 | 1 |  | 60 |
| GSM 2 + UMTS 1 | 2 |  | 30 | 0 |  | 0 | 0 |  | 0 | 1 |  | 60 |
| GSM 3 + UMTS 1 | 3 |  | 20 | 0 |  | 0 | 0 |  | 0 | 1 |  | 60 |
| GSM 4 + UMTS 1 | 4 |  | 15 | 0 |  | 0 | 0 |  | 0 | 1 |  | 60 |
| GSM 5 + UMTS 1 | 5 |  | 10 | 0 |  | 0 | 0 |  | 0 | 1 |  | 60 |
| GSM 6 + UMTS 1 | 6 |  | 7 | 0 |  | 0 | 0 |  | 0 | 1 |  | 60 |
| GSM 1 + UMTS 2 | 1 |  | 60 | 0 |  | 0 | 0 |  | 0 | 2 |  | 30 |
| GSM 2 + UMTS 2 | 2 |  | 30 | 0 |  | 0 | 0 |  | 0 | 2 |  | 30 |
| GSM 3 + UMTS 2 | 3 |  | 20 | 0 |  | 0 | 0 |  | 0 | 2 |  | 30 |
| GSM 4 + UMTS 2 | 4 |  | 15 | 0 |  | 0 | 0 |  | 0 | 2 |  | 30 |
| GSM 5 + UMTS 2 | 5 |  | 10 | 0 |  | 0 | 0 |  | 0 | 2 |  | 30 |
| GSM 6 + UMTS 2 | 6 |  | 7 | 0 |  | 0 | 0 |  | 0 | 2 |  | 30 |
| GSM 1 + UMTS 3 | 1 |  | 60 | 0 |  | 0 | 0 |  | 0 | 3 |  | 20 |
| GSM 2 + UMTS 3 | 2 |  | 30 | 0 |  | 0 | 0 |  | 0 | 3 |  | 20 |
| GSM 3 + UMTS 3 | 3 |  | 20 | 0 |  | 0 | 0 |  | 0 | 3 |  | 20 |
| GSM 4 + UMTS 3 | 4 |  | 15 | 0 |  | 0 | 0 |  | 0 | 3 |  | 20 |
| GSM 5 + UMTS 3 | 5 |  | 10 | 0 |  | 0 | 0 |  | 0 | 3 |  | 20 |
| GSM 2 + UMTS 4 | 2 |  | 30 | 0 |  | 0 | 0 |  | 0 | 4 |  | 15 |
| GSM 3 + UMTS 4 | 3 |  | 20 | 0 |  | 0 | 0 |  | 0 | 4 |  | 15 |
| GSM 4 + UMTS 4 | 4 |  | 15 | 0 |  | 0 | 0 |  | 0 | 4 |  | 15 |
| Table 11 Output power of the RRU3959a (GU MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of GSM TRXs |  |  | Total Number of UMTS Carriers |  |  | Output Power per GSM TRX (W) |  |  | Output Power per UMTS Carrier (W) |  |
| GSM + UMTS |  | 2 |  |  | 1 |  |  | 40 |  |  | 20 |  |
|  |  | 2 |  |  | 1 |  |  | 30 |  |  | 30 |  |
|  |  | 3 |  |  | 1 |  |  | 30 |  |  | 30 |  |
|  |  | 3 |  |  | 1 |  |  | 20 |  |  | 40 |  |
|  |  | 4 |  |  | 1 |  |  | 20 |  |  | 40 |  |
|  |  | 5 |  |  | 1 |  |  | 20 |  |  | 20 |  |
|  |  | 5 |  |  | 1 |  |  | 15 |  |  | 30 |  |
|  |  | 6 |  |  | 1 |  |  | 15 |  |  | 30 |  |
|  |  | 7 |  |  | 1 |  |  | 10 |  |  | 20 |  |
|  |  | 1 |  |  | 2 |  |  | 30 |  |  | 30 |  |
|  |  | 1 |  |  | 2 |  |  | 20 |  |  | 40 |  |
|  |  | 2 |  |  | 2 |  |  | 40 |  |  | 20 |  |
|  |  | 2 |  |  | 2 |  |  | 30 |  |  | 30 |  |
|  |  | 2 |  |  | 2 |  |  | 20 |  |  | 40 |  |
|  |  | 3 |  |  | 2 |  |  | 20 |  |  | 20 |  |
|  |  | 3 |  |  | 2 |  |  | 15 |  |  | 30 |  |
|  |  | 4 |  |  | 2 |  |  | 20 |  |  | 20 |  |
|  |  | 4 |  |  | 2 |  |  | 15 |  |  | 30 |  |
|  |  | 5 |  |  | 2 |  |  | 13 |  |  | 20 |  |
|  |  | 6 |  |  | 2 |  |  | 13 |  |  | 20 |  |
|  |  | 1 |  |  | 1 (MIMO) |  |  | 20 |  |  | 2x40 |  |
|  |  | 1 |  |  | 1 (MIMO) |  |  | 30 |  |  | 2x30 |  |
|  |  | 1 |  |  | 1 (MIMO) |  |  | 40 |  |  | 2x20 |  |
|  |  | 2 |  |  | 1 (MIMO) |  |  | 20 |  |  | 2x40 |  |
|  |  | 2 |  |  | 1 (MIMO) |  |  | 30 |  |  | 2x30 |  |
|  |  | 2 |  |  | 1 (MIMO) |  |  | 40 |  |  | 2x20 |  |
|  |  | 3 |  |  | 1 (MIMO) |  |  | 20 |  |  | 2x20 |  |
|  |  | 3 |  |  | 1 (MIMO) |  |  | 15 |  |  | 2x30 |  |
|  |  | 4 |  |  | 1 (MIMO) |  |  | 20 |  |  | 2x20 |  |
|  |  | 4 |  |  | 1 (MIMO) |  |  | 15 |  |  | 2x30 |  |
|  |  | 1 |  |  | 2 (MIMO) |  |  | 20 |  |  | 2x20 |  |
|  |  | 2 |  |  | 2 (MIMO) |  |  | 20 |  |  | 2x20 |  |
|  |  | 2 |  |  | 2 (MIMO) |  |  | 30 |  |  | 2x15 |  |
|  |  | 3 |  |  | 2 (MIMO) |  |  | 20 |  |  | 2x10 |  |
|  |  | 3 |  |  | 2 (MIMO) |  |  | 15 |  |  | 2x15 |  |
|  |  | 3 |  |  | 2 (MIMO) |  |  | 10 |  |  | 2x20 |  |
|  |  | 4 |  |  | 2 (MIMO) |  |  | 20 |  |  | 2x10 |  |
|  |  | 4 |  |  | 2 (MIMO) |  |  | 15 |  |  | 2x15 |  |
|  |  | 4 |  |  | 2 (MIMO) |  |  | 10 |  |  | 2x20 |  |

| Table 12 Configuration of RRU3959a PAs (GU MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) |  | PA1 |  |  |  |  |  | PA2 |  |  |  |  |  |
|  |  | Number of GSM TRXs | Output Power per GSM TRX (W) |  | Number of UMTS Carriers | Output Power per UMTS Carrier (W) |  | Number of GSM TRXs |  | Output Power per GSM TRX (W) | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |
| GSM 2 + UMTS 1 |  | 1 | 40 |  | 1 | 20 |  | 1 |  | 40 | 0 |  | 0 |
| GSM 2 + UMTS 1 |  | 1 | 30 |  | 1 | 30 |  | 1 |  | 30 | 0 |  | 0 |
| GSM 3 + UMTS 1 |  | 2 | 30 |  | 0 | 0 |  | 1 |  | 30 | 1 |  | 30 |
| GSM 3 + UMTS 1 |  | 2 | 20 |  | 0 | 0 |  | 1 |  | 20 | 1 |  | 40 |
| GSM 4 + UMTS 1 |  | 3 | 20 |  | 0 | 0 |  | 1 |  | 20 | 1 |  | 40 |
| GSM 5 + UMTS 1 |  | 3 | 20 |  | 0 | 0 |  | 2 |  | 20 | 1 |  | 20 |
| GSM 5 + UMTS 1 |  | 3 | 15 |  | 0 | 0 |  | 2 |  | 15 | 1 |  | 30 |
| GSM 6 + UMTS 1 |  | 4 | 15 |  | 0 | 0 |  | 2 |  | 15 | 1 |  | 30 |
| GSM 7 + UMTS 1 |  | 4 | 10 |  | 0 | 0 |  | 3 |  | 10 | 1 |  | 20 |
| GSM 1 + UMTS 2 |  | 1 | 30 |  | 1 | 30 |  | 0 |  | 0 | 1 |  | 30 |
| GSM 1 + UMTS 2 |  | 1 | 20 |  | 1 | 40 |  | 0 |  | 0 | 1 |  | 40 |
| GSM 2 + UMTS 2 |  | 1 | 40 |  | 1 | 20 |  | 1 |  | 40 | 1 |  | 20 |
| GSM 2 + UMTS 2 |  | 1 | 30 |  | 1 | 30 |  | 1 |  | 30 | 1 |  | 30 |
| GSM 2 + UMTS 2 |  | 1 | 20 |  | 1 | 40 |  | 1 |  | 20 | 1 |  | 40 |
| GSM 3 + UMTS 2 |  | 2 | 20 |  | 1 | 20 |  | 1 |  | 20 | 1 |  | 20 |
| GSM 3 + UMTS 2 |  | 2 | 15 |  | 1 | 30 |  | 1 |  | 15 | 1 |  | 30 |
| GSM 4 + UMTS 2 |  | 2 | 20 |  | 1 | 20 |  | 2 |  | 20 | 1 |  | 20 |
| GSM 4 + UMTS 2 |  | 2 | 15 |  | 1 | 30 |  | 2 |  | 15 | 1 |  | 30 |
| GSM 5 + UMTS 2 |  | 3 | 13 |  | 1 | 20 |  | 2 |  | 13 | 1 |  | 20 |
| GSM 6 + UMTS 2 |  | 3 | 13 |  | 1 | 20 |  | 3 |  | 13 | 1 |  | 20 |
| GSM 1 + UMTS 1 (MIMO) |  | 1 | 20 |  | 1 | 40 |  | 0 |  | 0 | 1 |  | 40 |
| GSM 1 + UMTS 1 (MIMO) |  | 1 | 30 |  | 1 | 30 |  | 0 |  | 0 | 1 |  | 30 |
| GSM 1 + UMTS 1 (MIMO) |  | 1 | 40 |  | 1 | 20 |  | 0 |  | 0 | 1 |  | 20 |
| GSM 2 + UMTS 1 (MIMO) |  | 1 | 20 |  | 1 | 40 |  | 1 |  | 20 | 1 |  | 40 |
| GSM 2 + UMTS 1 (MIMO) |  | 1 | 30 |  | 1 | 30 |  | 1 |  | 30 | 1 |  | 30 |
| GSM 2 + UMTS 1 (MIMO) |  | 1 | 40 |  | 1 | 20 |  | 1 |  | 40 | 1 |  | 20 |
| GSM 3 + UMTS 1 (MIMO) |  | 2 | 20 |  | 1 | 20 |  | 1 |  | 20 | 1 |  | 20 |
| GSM 3 + UMTS 1 (MIMO) |  | 2 | 15 |  | 1 | 30 |  | 1 |  | 15 | 1 |  | 30 |
| GSM 4 + UMTS 1 (MIMO) |  | 2 | 20 |  | 1 | 20 |  | 2 |  | 20 | 1 |  | 20 |
| GSM 4 + UMTS 1 (MIMO) |  | 2 | 15 |  | 1 | 30 |  | 2 |  | 15 | 1 |  | 30 |
| GSM 1 + UMTS 2 (MIMO) |  | 1 | 20 |  | 2 | 20 |  | 0 |  | 0 | 2 |  | 20 |
| GSM 2 + UMTS 2 (MIMO) |  | 1 | 20 |  | 2 | 20 |  | 1 |  | 20 | 2 |  | 20 |
| GSM 2 + UMTS 2 (MIMO) |  | 1 | 30 |  | 2 | 15 |  | 1 |  | 30 | 2 |  | 15 |
| GSM 3 + UMTS 2 (MIMO) |  | 2 | 20 |  | 2 | 10 |  | 1 |  | 20 | 2 |  | 10 |
| GSM 3 + UMTS 2 (MIMO) |  | 2 | 15 |  | 2 | 15 |  | 1 |  | 15 | 2 |  | 15 |
| GSM 3 + UMTS 2 (MIMO) |  | 2 | 10 |  | 2 | 20 |  | 1 |  | 10 | 2 |  | 20 |
| GSM 4 + UMTS 2 (MIMO) |  | 2 | 20 |  | 2 | 10 |  | 2 |  | 20 | 2 |  | 10 |
| GSM 4 + UMTS 2 (MIMO) |  | 2 | 15 |  | 2 | 15 |  | 2 |  | 15 | 2 |  | 15 |
| GSM 4 + UMTS 2 (MIMO) |  | 2 | 10 |  | 2 | 20 |  | 2 |  | 10 | 2 |  | 20 |
| Table 13 Output power of the RRU3959a (GL non-MSR, 1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT | Total Number of GSM TRXs |  |  | Total Number of LTE FDD Carriers |  |  | Output Power per GSM TRX (W) |  | Output Power per LTE FDD Carrier (W) |  |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| GSM + LTE FDD | 2 |  |  | 1 |  |  | 30 |  | 60 |  |  | 1.4, 3, 5, 10, 15, 20 |  |
|  | 3 |  |  | 1 |  |  | 20 |  | 60 |  |  | 1.4, 3, 5, 10, 15, 20 |  |
|  | 4 |  |  | 1 |  |  | 15 |  | 60 |  |  | 1.4, 3, 5, 10, 15, 20 |  |

| Table 14 Configuration of RRU3959a PAs (GL non-MSR, 1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of GSM TRXs |  | Output Power per GSM TRX (W) |  | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |  | Number of GSM TRXs | Output Power per GSM TRX (W) | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |
| GSM 2 + LTE FDD 1 | 2 |  | 30 |  | 0 | 0 |  | 0 | 0 | 1 |  | 60 |
| GSM 3 + LTE FDD 1 | 3 |  | 20 |  | 0 | 0 |  | 0 | 0 | 1 |  | 60 |
| GSM 4 + LTE FDD 1 | 4 |  | 15 |  | 0 | 0 |  | 0 | 0 | 1 |  | 60 |
| Table 15 Output power of the RRU3959a (GL MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of GSM TRXs |  | Total Number of LTE FDD Carriers |  |  | Output Power per GSM TRX (W) |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| GSM + LTE FDD |  | 1 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  |  | 15 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  |  | 15 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 5 |  | 1 (MIMO) |  |  | 7.5 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 5 |  | 1 (MIMO) |  |  | 12 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 6 |  | 1 (MIMO) |  |  | 7.5 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 6 |  | 1 (MIMO) |  |  | 12 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 2 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10 |  |
|  |  | 1 |  | 2 (MIMO) |  |  | 20 |  | - Carrier 1:2x30 - Carrier 2:2x10 |  | 1.4, 3, 5, 10 |  |
|  |  | 1 |  | 2 (MIMO) |  |  | 30 |  | - Carrier 1: 2x20 - Carrier 2: 2x10 |  | 1.4, 3, 5, 10 |  |
|  |  | 2 |  | 2 (MIMO) |  |  | 20 |  | - Carrier 1: 2x30 - Carrier 2: 2x10 |  | 1.4, 3, 5, 10 |  |
|  |  | 2 |  | 2 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10 |  |
|  |  | 3 |  | 2 (MIMO) |  |  | 10 |  | 2x20 |  | 1.4, 3, 5, 10 |  |
|  |  | 3 |  | 2 (MIMO) |  |  | 15 |  | - Carrier 1: 2x10 - Carrier 2: 2x20 |  | 1.4, 3, 5, 10 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 20 |  | 2x10 |  | 1.4, 3, 5, 10 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 10 |  | 2x20 |  | 1.4, 3, 5, 10 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 15 |  | - Carrier 1: 2x10 - Carrier 2: 2x20 |  | 1.4, 3, 5, 10 |  |

| Table 16 Configuration of RRU3959a PAs (GL MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of GSM TRXs |  | Output Power per GSM TRX (W) |  | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |  | Number of GSM TRXs | Output Power per GSM TRX (W) | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 0 | 0 | 1 |  | 30 |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 0 | 0 | 1 |  | 40 |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 0 | 0 | 1 |  | 20 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 1 | 20 | 1 |  | 40 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 1 | 30 | 1 |  | 30 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 1 | 40 | 1 |  | 20 |
| GSM 3 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 1 | 20 | 1 |  | 20 |
| GSM 3 + LTE FDD 1 (MIMO) | 2 |  | 15 |  | 1 | 30 |  | 2 | 15 | 1 |  | 30 |
| GSM 4 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 2 | 20 | 1 |  | 20 |
| GSM 4 + LTE FDD 1 (MIMO) | 2 |  | 15 |  | 1 | 30 |  | 2 | 15 | 1 |  | 30 |
| GSM 5 + LTE FDD 1 (MIMO) | 3 |  | 7.5 |  | 1 | 30 |  | 2 | 7.5 | 1 |  | 30 |
| GSM 5 + LTE FDD 1 (MIMO) | 3 |  | 12 |  | 1 | 20 |  | 2 | 12 | 1 |  | 20 |
| GSM 6 + LTE FDD 1 (MIMO) | 3 |  | 7.5 |  | 1 | 30 |  | 3 | 7.5 | 1 |  | 30 |
| GSM 6 + LTE FDD 1 (MIMO) | 3 |  | 12 |  | 1 | 20 |  | 3 | 12 | 1 |  | 20 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | 20 |  | 0 | 0 | 2 |  | 20 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | - Carrier 1: 30 - Carrier 2: 10 |  | 0 | 0 | 2 |  | - Carrier 1: 30 - Carrier 2: 10 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 30 |  | 2 | - Carrier 1: 20 - Carrier 2: 10 |  | 0 | 0 | 2 |  | - Carrier 1: 20 - Carrier 2: 10 |
| GSM 2 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | - Carrier 1: 30 - Carrier 2: 10 |  | 1 | 20 | 2 |  | - Carrier 1: 30 - Carrier 2: 10 |
| GSM 2 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | 20 |  | 1 | 20 | 2 |  | 20 |
| GSM 3 + LTE FDD 2 (MIMO) | 2 |  | 10 |  | 2 | 20 |  | 1 | 10 | 2 |  | 20 |
| GSM 3 + LTE FDD 2 (MIMO) | 2 |  | 15 |  | 2 | - Carrier 1: 10 - Carrier 2: 20 |  | 1 | 15 | 2 |  | - Carrier 1: 10 - Carrier 2: 20 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 20 |  | 2 | 10 |  | 2 | 20 | 2 |  | 10 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 10 |  | 2 | 20 |  | 2 | 10 | 2 |  | 20 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 15 |  | 2 | - Carrier 1: 10 - Carrier 2: 20 |  | 2 | 15 | 2 |  | - Carrier 1: 10 - Carrier 2: 20 |
| Table 17 Output power of the RRU3959a (GL MSR, 1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of GSM TRXs |  | Total Number of LTE FDD Carriers |  |  | Output Power per GSM TRX (W) |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| GSM + LTE FDD |  | 1 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  |  | 15 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 5 |  | 1 (MIMO) |  |  | 7.5 |  | 2x30 |  | 10 |  |
|  |  | 5 |  | 1 (MIMO) |  |  | 12 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 6 |  | 1 (MIMO) |  |  | 7.5 |  | 2x30 |  | 10 |  |
|  |  | 6 |  | 1 (MIMO) |  |  | 12 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 2 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10 |  |
|  |  | 1 |  | 2 (MIMO) |  |  | 20 |  | - Carrier 1: 2x30 - Carrier 2: 2x10 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 1 |  | 2 (MIMO) |  |  | 30 |  | - Carrier 1: 2x20 - Carrier 2: 2x10 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 2 |  | 2 (MIMO) |  |  | 20 |  | - Carrier 1: 2x30 - Carrier 2: 2x10 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 2 |  | 2 (MIMO) |  |  | 20 |  | 2x20 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 3 |  | 2 (MIMO) |  |  | 10 |  | 2x20 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 3 |  | 2 (MIMO) |  |  | 15 |  | - Carrier 1: 2x10 - Carrier 2: 2x20 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 20 |  | 2x10 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 10 |  | 2x20 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 15 |  | - Carrier 1: 2x10 - Carrier 2: 2x20 |  | - Carrier 1: 1.4, 3, 5, 10, 15, 20 - Carrier 2: 1.4, 3, 5, 10 |  |

| Table 18 Configuration of RRU3959a PAs (GL MSR, 1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of GSM TRXs |  | Output Power per GSM TRX (W) |  | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |  | Number of GSM TRXs | Output Power per GSM TRX (W) | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 0 | 0 | 1 |  | 30 |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 0 | 0 | 1 |  | 40 |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 0 | 0 | 1 |  | 20 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 1 | 20 | 1 |  | 40 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 1 | 30 | 1 |  | 30 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 1 | 40 | 1 |  | 20 |
| GSM 3 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 1 | 20 | 1 |  | 20 |
| GSM 4 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 2 | 20 | 1 |  | 20 |
| GSM 4 + LTE FDD 1 (MIMO) | 2 |  | 15 |  | 1 | 30 |  | 2 | 15 | 1 |  | 30 |
| GSM 5 + LTE FDD 1 (MIMO) | 3 |  | 7.5 |  | 1 | 30 |  | 2 | 7.5 | 1 |  | 30 |
| GSM 5 + LTE FDD 1 (MIMO) | 3 |  | 12 |  | 1 | 20 |  | 2 | 12 | 1 |  | 20 |
| GSM 6 + LTE FDD 1 (MIMO) | 3 |  | 7.5 |  | 1 | 30 |  | 3 | 7.5 | 1 |  | 30 |
| GSM 6 + LTE FDD 1 (MIMO) | 3 |  | 12 |  | 1 | 20 |  | 3 | 12 | 1 |  | 20 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | 20 |  | 0 | 0 | 2 |  | 20 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | - Carrier 1: 30 - Carrier 2: 10 |  | 0 | 0 | 2 |  | - Carrier 1: 30 - Carrier 2: 10 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 30 |  | 2 | - Carrier 1: 20 - Carrier 2: 10 |  | 0 | 0 | 2 |  | - Carrier 1: 20 - Carrier 2: 10 |
| GSM 2 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | - Carrier 1: 30 - Carrier 2: 10 |  | 1 | 20 | 2 |  | - Carrier 1: 30 - Carrier 2: 10 |
| GSM 2 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | 20 |  | 1 | 20 | 2 |  | 20 |
| GSM 3 + LTE FDD 2 (MIMO) | 2 |  | 10 |  | 2 | 20 |  | 1 | 10 | 2 |  | 20 |
| GSM 3 + LTE FDD 2 (MIMO) | 2 |  | 15 |  | 2 | - Carrier 1: 10 - Carrier 2: 20 |  | 1 | 15 | 2 |  | - Carrier 1: 10 - Carrier 2: 20 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 20 |  | 2 | 10 |  | 2 | 20 | 2 |  | 10 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 10 |  | 2 | 20 |  | 2 | 10 | 2 |  | 20 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 15 |  | 2 | - Carrier 1: 10 - Carrier 2: 20 |  | 2 | 15 | 2 |  | - Carrier 1: 10 - Carrier 2: 20 |
| Table 19 Output power of the RRU3959a (UL MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of UMTS Carriers |  | Total Number of LTE FDD Carriers |  |  | Output Power per UMTS Carrier (W) |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| UMTS + LTE FDD |  | 1 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |

| Table 20 Configuration of RRU3959a PAs (UL MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |  | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |  | Number of UMTS Carriers | Output Power per UMTS Carrier (W) | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |
| UMTS 1 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 0 | 0 | 1 |  | 30 |
| UMTS 1 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 0 | 0 | 1 |  | 20 |
| UMTS 1 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 0 | 0 | 1 |  | 40 |
| UMTS 2 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 1 | 30 | 1 |  | 30 |
| UMTS 2 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 1 | 40 | 1 |  | 20 |
| UMTS 2 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 1 | 20 | 1 |  | 40 |
| UMTS 3 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 1 | 20 | 1 |  | 20 |
| UMTS 4 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 2 | 20 | 1 |  | 20 |
| Table 21 Output power of the RRU3959a (UL MSR, 2100 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of UMTS Carriers |  | Total Number of LTE FDD Carriers |  |  | Output Power per UMTS Carrier (W) |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| UMTS + LTE FDD |  | 1 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 1 (MIMO) |  | 1 (MIMO) |  |  | 2x30 |  | 2x30 |  | 5, 10, 15, 20 |  |
|  |  | 2 (MIMO) |  | 1 (MIMO) |  |  | 2x20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 3 (MIMO) |  | 1 (MIMO) |  |  | 2x15 |  | 2x15 |  | 5, 10, 15, 20 |  |
|  |  | 6 |  | 1 (MIMO) |  |  | 15 |  | 2x15 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 2 (MIMO) |  |  | 20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 15 |  | 2x15 |  | 5, 10, 15, 20 |  |

| Table 22 Configuration of RRU3959a PAs (UL MSR, 2100 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  |  | PA2 |  |  |  |  |  |  |
|  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |  | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |  | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |
| UMTS 1 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 |  | 30 |  | 0 |  | 0 |  | 1 |  | 30 |
| UMTS 2 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 |  | 30 |  | 1 |  | 30 |  | 1 |  | 30 |
| UMTS 3 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 |  | 20 |  | 1 |  | 20 |  | 1 |  | 20 |
| UMTS 4 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 |  | 20 |  | 2 |  | 20 |  | 1 |  | 20 |
| UMTS 1 (MIMO) + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 |  | 30 |  | 1 |  | 30 |  | 1 |  | 30 |
| UMTS 2 (MIMO) + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 |  | 20 |  | 2 |  | 20 |  | 1 |  | 20 |
| UMTS 3 (MIMO) + LTE FDD 1 (MIMO) | 3 |  | 15 |  | 1 |  | 15 |  | 3 |  | 15 |  | 1 |  | 15 |
| UMTS 6 + LTE FDD 1 (MIMO) | 3 |  | 15 |  | 1 |  | 15 |  | 3 |  | 15 |  | 1 |  | 15 |
| UMTS 2 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 |  | 20 |  | 1 |  | 20 |  | 2 |  | 20 |
| UMTS 4 + LTE FDD 2 (MIMO) | 2 |  | 15 |  | 2 |  | 15 |  | 2 |  | 15 |  | 2 |  | 15 |
| Table 23 Output power of the RRU3959a (GUL MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of GSM TRXs |  | Total Number of UMTS Carriers |  | Total Number of LTE FDD Carriers |  | Output Power per GSM TRX (W) |  | Output Power per UMTS Carrier (W) |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| GSM + UMTS + LTE FDD |  | 1 |  | 1 |  | 1 (MIMO) |  | 20 |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 |  | 1 (MIMO) |  | 20 |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 1 |  | 2 |  | 1 (MIMO) |  | 40 |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 |  | 2 |  | 1 (MIMO) |  | 20 |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |  |

| Table 24 Configuration of RRU3959a PAs (GUL MSR, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  | PA2 |  |  |  |  |  |  |
|  | Number of GSM TRXs | Output Power per GSM TRX (W) | Number of UMTS Carriers | Output Power per UMTS Carrier (W) |  | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) | Number of GSM TRXs | Output Power per GSM TRX (W) |  | Number of UMTS Carriers | Output Power per UMTS Carrier (W) | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |
| GSM 1 + UMTS 1 + LTE FDD 1 (MIMO) | 1 | 20 | 0 | 0 |  | 1 | 40 | 0 | 0 |  | 1 | 20 | 1 | 40 |
| GSM 2 + UMTS 1 + LTE FDD 1 (MIMO) | 2 | 20 | 0 | 0 |  | 1 | 20 | 0 | 0 |  | 1 | 20 | 1 | 20 |
| GSM 1 + UMTS 2 + LTE FDD 1 (MIMO) | 1 | 40 | 0 | 0 |  | 1 | 20 | 0 | 0 |  | 2 | 20 | 1 | 20 |
| GSM 2 + UMTS 2 + LTE FDD 1 (MIMO) | 2 | 20 | 0 | 0 |  | 1 | 20 | 0 | 0 |  | 2 | 20 | 1 | 20 |
| Table 25 Output power of the RRU3959a (LTE NB-IoT, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  |  |  |  | Total Number of LTE NB-IoT Carriers |  |  |  |  | Output Power per LTE NB-IoT Carrier (W) |  |  |  |  |
| LTE NB-IoT |  |  |  |  | 1 |  |  |  |  | 2x20 |  |  |  |  |

| Table 26 Configuration of RRU3959a PAs (LTE NB-IoT, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) |  | PA1 |  |  | PA2 |  |  |  |
|  |  | Number of LTE NB-IoT Carriers | Output Power per LTE NB-IoT Carrier (W) |  | Number of LTE NB-IoT Carriers |  | Output Power per LTE NB-IoT Carrier (W) |  |
| LTE NB-IoT 1 |  | 1 | 20 |  | 1 |  | 20 |  |
| Table 27 Output power of the RRU3959a (GM, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |
| RAT | Total Number of GSM TRXs |  |  | Total Number of LTE NB-IoT Carriers |  | Output Power per GSM TRX (W) |  | Output Power per LTE NB-IoT Carrier (W) |
| GSM + LTE NB-IoT | 1 |  |  | 1 |  | 30 |  | 2x10 |
|  | 2 |  |  | 1 |  | 30 |  | 2x10 |
|  | 3 |  |  | 1 |  | 20 |  | 2x6.7 |
|  | 4 |  |  | 1 |  | 20 |  | 2x6.7 |
|  | 5 |  |  | 1 |  | 15 |  | 2x5 |
|  | 6 |  |  | 1 |  | 15 |  | 2x5 |
|  | 1 |  |  | 1 |  | 50 |  | 2x10 |
|  | 2 |  |  | 1 |  | 50 |  | 2x10 |
|  | 3 |  |  | 1 |  | 27.3 |  | 2x5.4 |
|  | 4 |  |  | 1 |  | 27.3 |  | 2x5.4 |
|  | 5 |  |  | 1 |  | 18.8 |  | 2x3.6 |
|  | 6 |  |  | 1 |  | 18.8 |  | 2x3.6 |

| Table 28 Configuration of RRU3959a PAs (GM, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  | PA2 |  |  |  |  |  |
|  | Number of GSM TRXs |  | Output Power per GSM TRX (W) | Number of LTE NB-IoT Carriers |  | Output Power per LTE NB-IoT Carrier (W) | Number of GSM TRXs |  | Output Power per GSM TRX (W) | Number of LTE NB-IoT Carriers |  | Output Power per LTE NB-IoT Carrier (W) |
| GSM 1 + LTE NB-IoT 1 | 1 |  | 30 | 1 |  | 10 | 0 |  | 0 | 1 |  | 10 |
| GSM 2 + LTE NB-IoT 1 | 1 |  | 30 | 1 |  | 10 | 1 |  | 30 | 1 |  | 10 |
| GSM 3 + LTE NB-IoT 1 | 2 |  | 20 | 1 |  | 6.7 | 1 |  | 20 | 1 |  | 6.7 |
| GSM 4 + LTE NB-IoT 1 | 2 |  | 20 | 1 |  | 6.7 | 2 |  | 20 | 1 |  | 6.7 |
| GSM 5 + LTE NB-IoT 1 | 3 |  | 15 | 1 |  | 5 | 2 |  | 15 | 1 |  | 5 |
| GSM 6 + LTE NB-IoT 1 | 3 |  | 15 | 1 |  | 5 | 3 |  | 15 | 1 |  | 5 |
| GSM 1 + LTE NB-IoT 1 | 1 |  | 50 | 1 |  | 10 | 0 |  | 50 | 1 |  | 10 |
| GSM 2 + LTE NB-IoT 1 | 1 |  | 50 | 1 |  | 10 | 1 |  | 50 | 1 |  | 10 |
| GSM 3 + LTE NB-IoT 1 | 2 |  | 27.3 | 1 |  | 5.4 | 1 |  | 27.3 | 1 |  | 5.4 |
| GSM 4 + LTE NB-IoT 1 | 2 |  | 27.3 | 1 |  | 5.4 | 2 |  | 27.3 | 1 |  | 5.4 |
| GSM 5 + LTE NB-IoT 1 | 3 |  | 18.8 | 1 |  | 3.6 | 2 |  | 18.8 | 1 |  | 3.6 |
| GSM 6 + LTE NB-IoT 1 | 3 |  | 18.8 | 1 |  | 3.6 | 3 |  | 18.8 | 1 |  | 3.6 |
| Table 29 Output power of the RRU3959a (UM, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of UMTS Carriers |  |  | Total Number of LTE NB-IoT Carriers |  |  | Output Power per UMTS Carrier (W) |  |  | Output Power per LTE NB-IoT Carrier (W) |  |
| UMTS + LTE NB-IoT |  | 1 |  |  | 1 |  |  | 45 |  |  | 2x15 |  |
|  |  | 2 |  |  | 1 |  |  | 25.7 |  |  | 2x8.6 |  |
|  |  | 3 |  |  | 1 |  |  | 18 |  |  | 2x6 |  |

| Table 30 Configuration of RRU3959a PAs (UM, 900 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |  | Number of LTE NB-IoT Carriers | Output Power per LTE NB-IoT Carrier (W) |  | Number of UMTS Carriers | Output Power per UMTS Carrier (W) | Number of LTE NB-IoT Carriers |  | Output Power per LTE NB-IoT Carrier (W) |
| UMTS 1 + LTE NB-IoT 1 | 1 |  | 45 |  | 1 | 15 |  | 0 | 0 | 1 |  | 15 |
| UMTS 2 + LTE NB-IoT 1 | 1 |  | 25.7 |  | 1 | 8.6 |  | 1 | 25.7 | 1 |  | 8.6 |
| UMTS 3 + LTE NB-IoT 1 | 2 |  | 18 |  | 1 | 6 |  | 1 | 18 | 1 |  | 6 |
| Table 31 Output power of the RRU3959a (LM, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| RAT |  | Total Number of LTE FDD Carriers |  | Total Number of LTE NB-IoT Carriers |  |  | Output Power per LTE FDD Carrier (W) |  | Output Power per LTE NB-IoT Carrier (W) |  | Bandwidth per LTE FDD Carrier (MHz) |  |
| LTE FDD + LTE NB-IoT |  | 1 (MIMO) |  | 1 |  |  | 2x40 |  | 2x10 |  | 5 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x40 |  | 2x5 |  | 10 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x40 |  | 2x5 |  | 15 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x40 |  | 2x3.2 |  | 20 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x45 |  | 2x15 |  | 5 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x51.4 |  | 2x8.6 |  | 10 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x54 |  | 2x6 |  | 15 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x55.4 |  | 2x4.6 |  | 20 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x40 |  | 2x18.8 |  | 5 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x50.4 |  | 2x9.6 |  | 10 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x53.6 |  | 2x6.4 |  | 15 |  |
|  |  | 1 (MIMO) |  | 1 |  |  | 2x55.2 |  | 2x4.8 |  | 20 |  |

| Table 32 Configuration of RRU3959a PAs (LM, 900 MHz/1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  |  | PA2 |  |  |  |  |  |  |
|  | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |  | Number of LTE NB-IoT Carriers |  | Output Power per LTE NB-IoT Carrier (W) |  | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |  | Number of LTE NB-IoT Carriers |  | Output Power per LTE NB-IoT Carrier (W) |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 40 |  | 1 |  | 10 |  | 1 |  | 40 |  | 1 |  | 10 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 40 |  | 1 |  | 5 |  | 1 |  | 40 |  | 1 |  | 5 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 40 |  | 1 |  | 5 |  | 1 |  | 40 |  | 1 |  | 5 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 40 |  | 1 |  | 3.2 |  | 1 |  | 40 |  | 1 |  | 3.2 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 45 |  | 1 |  | 15 |  | 1 |  | 45 |  | 1 |  | 15 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 51.4 |  | 1 |  | 8.6 |  | 1 |  | 51.4 |  | 1 |  | 8.6 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 54 |  | 1 |  | 6 |  | 1 |  | 54 |  | 1 |  | 6 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 55.4 |  | 1 |  | 4.6 |  | 1 |  | 55.4 |  | 1 |  | 4.6 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 40 |  | 1 |  | 18.8 |  | 1 |  | 40 |  | 1 |  | 18.8 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 50.4 |  | 1 |  | 9.6 |  | 1 |  | 50.4 |  | 1 |  | 9.6 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 53.6 |  | 1 |  | 6.4 |  | 1 |  | 53.6 |  | 1 |  | 6.4 |
| LTE FDD 1 (MIMO) + LTE NB-IoT 1 | 1 |  | 55.2 |  | 1 |  | 4.8 |  | 1 |  | 55.2 |  | 1 |  | 4.8 |
| Table 33 Output power for the RRU3959a (GLM, 1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Working Mode |  | Total Number of GSM Carriers |  | Total Number of LTE FDD Carriers |  | Total Number of LTE (NB-IoT) Carriers |  | Output Power per GSM Carrier (W) |  | Output Power per LTE FDD Carrier (W) |  | Output Power per LTE (NB-IoT) Carrier (W) |  | Bandwidth of LTE FDD Carrier (MHz) |  |
| GSM + LTE FDD + LTE (NB-IoT) |  | 1 |  | 1 (MIMO) |  | 1 |  | 30 |  | 2x20 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x30 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 25 |  | 2x25 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 30 |  | 2x25 |  | 2x3.5 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 25 |  | 2x30 |  | 2x3.5 |  | 10 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 30 |  | 2x20 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x30 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 25 |  | 2x25 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 30 |  | 2x25 |  | 2x3.5 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 25 |  | 2x30 |  | 2x3.5 |  | 10 |  |
|  |  | 3 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x10 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  | 1 |  | 15 |  | 2x20 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  | 1 |  | 13 |  | 2x24 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x10 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  | 1 |  | 15 |  | 2x20 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  | 1 |  | 13 |  | 2x24 |  | 2x10 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 35 |  | 2x20 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 30 |  | 2x25 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 25 |  | 2x30 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 1 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x35 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 35 |  | 2x20 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 30 |  | 2x25 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 25 |  | 2x30 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x35 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x15 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  | 1 |  | 17 |  | 2x20 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  | 1 |  | 15 |  | 2x25 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  | 1 |  | 13 |  | 2x26 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  | 1 |  | 20 |  | 2x15 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  | 1 |  | 17 |  | 2x20 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  | 1 |  | 15 |  | 2x25 |  | 2x5 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  | 1 |  | 13 |  | 2x26 |  | 2x5 |  | 5, 10, 15, 20 |  |

| Table 34 Configuration of RRU3959a PAs (GLM, 1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario | PA1 |  |  |  |  |  | PA2 |  |  |  |  |  |
|  | Number of GSM Carriers | Output Power per GSM Carrier (W) | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) | Number of LTE (NB-IoT) Carriers | Output Power per LTE (NB-IoT) Carrier (W) | Number of GSM Carriers | Output Power per GSM Carrier (W) | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) | Number of LTE (NB-IoT) Carriers | Output Power per LTE (NB-IoT) Carrier (W) |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 30 | 1 | 20 | 1 | 10 | 0 | 0 | 1 | 20 | 1 | 10 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 20 | 1 | 30 | 1 | 10 | 0 | 0 | 1 | 30 | 1 | 10 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 25 | 1 | 25 | 1 | 10 | 0 | 0 | 1 | 25 | 1 | 10 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 30 | 1 | 25 | 1 | 3.5 | 0 | 0 | 1 | 25 | 1 | 3.5 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 25 | 1 | 30 | 1 | 3.5 | 0 | 0 | 1 | 30 | 1 | 3.5 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 30 | 1 | 20 | 1 | 10 | 1 | 30 | 1 | 20 | 1 | 10 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 20 | 1 | 30 | 1 | 10 | 1 | 20 | 1 | 30 | 1 | 10 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 25 | 1 | 25 | 1 | 10 | 1 | 25 | 1 | 25 | 1 | 10 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 30 | 1 | 25 | 1 | 3.5 | 1 | 30 | 1 | 25 | 1 | 3.5 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 25 | 1 | 30 | 1 | 3.5 | 1 | 25 | 1 | 30 | 1 | 3.5 |
| GSM 3 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 20 | 1 | 10 | 1 | 10 | 1 | 20 | 1 | 10 | 1 | 10 |
| GSM 3 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 15 | 1 | 20 | 1 | 10 | 1 | 15 | 1 | 20 | 1 | 10 |
| GSM 3 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 13 | 1 | 24 | 1 | 10 | 1 | 13 | 1 | 24 | 1 | 10 |
| GSM 4 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 20 | 1 | 10 | 1 | 10 | 2 | 20 | 1 | 10 | 1 | 10 |
| GSM 4 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 15 | 1 | 20 | 1 | 10 | 2 | 15 | 1 | 20 | 1 | 10 |
| GSM 4 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 13 | 1 | 24 | 1 | 10 | 2 | 13 | 1 | 24 | 1 | 10 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 35 | 1 | 20 | 1 | 5 | 0 | 0 | 1 | 20 | 1 | 5 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 30 | 1 | 25 | 1 | 5 | 0 | 0 | 1 | 25 | 1 | 5 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 25 | 1 | 30 | 1 | 5 | 0 | 0 | 1 | 30 | 1 | 5 |
| GSM 1 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 20 | 1 | 35 | 1 | 5 | 0 | 0 | 1 | 35 | 1 | 5 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 35 | 1 | 20 | 1 | 5 | 1 | 35 | 1 | 20 | 1 | 5 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 30 | 1 | 25 | 1 | 5 | 1 | 30 | 1 | 25 | 1 | 5 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 25 | 1 | 30 | 1 | 5 | 1 | 25 | 1 | 30 | 1 | 5 |
| GSM 2 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 1 | 20 | 1 | 35 | 1 | 5 | 1 | 20 | 1 | 35 | 1 | 5 |
| GSM 3 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 20 | 1 | 15 | 1 | 5 | 1 | 20 | 1 | 15 | 1 | 5 |
| GSM 3 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 17 | 1 | 20 | 1 | 5 | 1 | 17 | 1 | 20 | 1 | 5 |
| GSM 3 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 15 | 1 | 25 | 1 | 5 | 1 | 15 | 1 | 25 | 1 | 5 |
| GSM 3 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 13 | 1 | 26 | 1 | 5 | 1 | 13 | 1 | 26 | 1 | 5 |
| GSM 4 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 20 | 1 | 15 | 1 | 5 | 2 | 20 | 1 | 15 | 1 | 5 |
| GSM 4 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 17 | 1 | 20 | 1 | 5 | 2 | 17 | 1 | 20 | 1 | 5 |
| GSM 4 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 15 | 1 | 25 | 1 | 5 | 2 | 15 | 1 | 25 | 1 | 5 |
| GSM 4 + LTE (FDD)1 (MIMO) + LTE (NB-IoT) 1 | 2 | 13 | 1 | 26 | 1 | 5 | 2 | 13 | 1 | 26 | 1 | 5 |

Parent Topic: Typical Power Configurations for RRUs

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >