# RRU5909s Hardware Description

## Contents

6.3.5.1 RRU5909s Hardware Description

6.3.5.1.1 Changes in RRU5909s Hardware Description

6.3.5.1.2 RRU Introduction

6.3.5.1.2.1 RRU Exterior

6.3.5.1.2.2 RRU Functions

6.3.5.1.2.3 RRU Technical Specifications

6.3.5.1.2.4 RRU Ports

6.3.5.1.2.5 RRU Indicators

6.3.5.1.2.6 Optical Modules

6.3.5.1.3 RRU Cables

6.3.5.1.3.1 RRU Cable List

6.3.5.1.3.2 RRU PGND Cable

6.3.5.1.3.3 RRU Power Cable

6.3.5.1.3.4 CPRI Fiber Optic Cable

6.3.5.1.3.5 RRU RF Jumper

6.3.5.1.3.6 RRU AISG Multi-Wire Cable

6.3.5.1.3.7 RRU AISG Extension Cable

6.3.5.1.4 RF Cable Connections for the RRU5909s

6.3.5.1.5 RRU Auxiliary Devices

# 6.3.5.1 RRU5909s Hardware Description

#### Purpose

This document provides reference for planning and deploying a DC blade RRU5909s (referred to as RRU in this document). RRU is short for remote radio unit. It presents the exterior and describes the ports, functions, cable types, connector specifications, and cable connections of the RRU.

#### Product Versions

The following table lists the product versions related to the RRU.

| Product Name | Solution Version | Product Version |
| --- | --- | --- |
| RRU5909s | - SRAN10.1 and later versions - GBSS17.1 and later versions - RAN17.1 and later versions - eRAN8.1 and later versions | V100R010C10 and later versions For details about the software versions of RF modules, see Software and Hardware Compatibility Description. |

#### Intended Audience

This document is intended for:

- Base station installation engineers

- System engineers

- Site maintenance engineers

#### Organization

Changes in RRU5909s Hardware Description

This section describes the changes in RRU5909s Hardware Description.

RRU Introduction

This chapter describes the exterior and function of the RRU as well as the ports and indicators on the RRU.

RRU Cables

This chapter describes RRU cables.

RF Cable Connections for the RRU5909s

RF cable connections for the RRU vary depending on the configurations of the RRU and antenna.

RRU Auxiliary Devices

This chapter describes RRU auxiliary devices.

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

Next topic >

# 6.3.5.1.1 Changes in RRU5909s Hardware Description

This section describes the changes in RRU5909s Hardware Description.

#### 03 (2017-09-30)

This is the third official release.

Compared with Issue 02 (2017-06-15), this issue does not include any new information.

Compared with Issue 02 (2017-06-15), this issue includes the following changes.

| Topic | Change Description |
| --- | --- |
| About This Document | Updated the required product versions and added support of the RRU by the DBS5900. |

Compared with Issue 02 (2017-06-15), no information is deleted from this issue.

#### 02 (2017-06-15)

This is the second official release.

Compared with Issue 01 (2017-05-20), this issue does not include any new information.

Compared with Issue 01 (2017-05-20), this issue includes the following changes.

| Topic | Change Description |
| --- | --- |
| RRU Functions | Added descriptions of the OPM50M(Ver.B). |
| RRU Auxiliary Devices |  |

Compared with Issue 01 (2017-05-20), no information is deleted from this issue.

#### 01 (2017-05-20)

This is the first official release.

Compared with Draft C (2017-03-30), this issue does not include any new information.

Compared with Draft C (2017-03-30), this issue does not include any changes.

Compared with Draft C (2017-03-30), no information is deleted from this issue.

#### Draft C (2017-03-30)

This is a draft.

Compared with Draft B (2017-03-10), this issue does not include any new information.

Compared with Draft B (2017-03-10), this issue includes the following changes.

| Topic | Change Description |
| --- | --- |
| RRU Power Cable | Updated the specifications of an RRU power cable. |

Compared with Draft B (2017-03-10), no information is deleted from this issue.

#### Draft B (2017-03-10)

This is a draft.

Compared with Draft A (2017-01-20), this issue does not include any new information.

Compared with Draft A (2017-01-20), this issue includes the following changes.

| Topic | Change Description |
| --- | --- |
| RRU Functions | Optimized the contents in this document. No technical change is involved. |

Compared with Draft A (2017-01-20), no information is deleted from this issue.

#### Draft A (2017-01-20)

This is a draft.

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

Next topic >

# 6.3.5.1.2 RRU Introduction

This chapter describes the exterior and function of the RRU as well as the ports and indicators on the RRU.

RRU Exterior

This section describes the exterior and dimensions of an RRU.

RRU Functions

This section describes main functions of an RRU.

RRU Technical Specifications

This section describes technical specifications of an RRU, including supported modes, frequency bands, RF specifications, engineering specifications, and antenna capabilities.

RRU Ports

This section describes ports on the RRU panels. An RRU has a bottom panel, cabling cavity panel, and indicator panel.

RRU Indicators

This section describes six indicators on an RRU. They indicate the running status of the RRU.

Optical Modules

An optical module transmits optical signals between an optical port and a fiber optic cable.

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.2.1 RRU Exterior

This section describes the exterior and dimensions of an RRU.

Figure 1 shows the exterior of an RRU.

Figure 1 RRU exterior

Figure 2 shows RRU dimensions.

Figure 2 RRU dimensions

You can obtain the RRU name, RRU frequency band and power supply information from the nameplate on the cover plate. Figure 3 shows the positions of the nameplate on the RRU.

NOTE:

The actual nameplate may differ from what is shown in the figure.

Figure 3 Positions of the nameplate

| (1) Nameplate | (2) Module name | (3) Frequency band | (4) Power supply type |
| --- | --- | --- | --- |

Parent Topic: RRU Introduction

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

Next topic >

# 6.3.5.1.2.2 RRU Functions

This section describes main functions of an RRU.

The Remote Radio Unit (RRU) consists of the high-speed interface unit, signal processing unit, power amplifier, duplexer, extension ports, and power module.

Figure 1 shows the functional structure of the RRU.

Figure 1 Functional structure of an RRU

NOTE:

For details about the extension ports, see RRU Ports.

The RRU performs the following functions:

- Receives downlink baseband data from the BBU and sends uplink baseband data to the BBU.

- Receives RF signals from the antenna system, down-converts the signals to intermediate frequency (IF) signals, amplifies the IF signals, performs analog-to-digital conversion, and up-converts RF signals to the transmit (TX) band.

- Multiplexes receive (RX) and TX signals on the RF channel so that these signals can share the same antenna channel, and filters the RX and TX signals.

- Provides a built-in Bias Tee (BT). The built-in BT couples RF signals and OOK signals and transmits them through the TX/RX port A. The built-in BT also supplies power to the tower mounted amplifier (TMA).

- The RRU can work with the Monitoring Extension Box (MEB). The MEB can transmit external RS-485 communication signals and dry node monitoring signals into the RRU, implementing information monitoring and reporting. For details about the MEB, see MEB06D User Guide.

- When an RRU is equipped with an AC/DC power module, the RRU and the power module together are called an AC RRU. For details about the AC/DC power module, see OPM15M(Ver.B) User Guide or OPM50M(Ver.B) User Guide.

Parent Topic: RRU Introduction

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.2.3 RRU Technical Specifications

This section describes technical specifications of an RRU, including supported modes, frequency bands, RF specifications, engineering specifications, and antenna capabilities.

For details about technical specifications of an RRU, see section Technical Specifications of RRUs in the Base Station Technical Description.

Parent Topic: RRU Introduction

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.2.4 RRU Ports

This section describes ports on the RRU panels. An RRU has a bottom panel, cabling cavity panel, and indicator panel.

Figure 1 shows the ports on the RRU panels.

Figure 1 Ports on the RRU panels

Table 1 describes ports and indicators on the RRU panels.

| Table 1 Ports and indicators on the RRU panels |  |  |
| --- | --- | --- |
| Item | Silkscreen | Remarks |
| (1) Ports in the cabling cavity | RTN(+) | Power supply socket, For details about RRU power cable appearance and specifications, see RRU Power Cable. |
|  | NEG(-) |  |
|  | CPRI0 | Optical/electrical port 0, connected to the BBU or an upper-level RRU |
|  | CPRI1 | Optical/electrical port 1, connected to a lower-level RRU or the BBU |
| (2) Bottom ports | A T/R | TX/RX port A (4.3-10 female connector), supporting RET signal transmission |
|  | B T/R | TX/RX port B (4.3-10 femalet connector) |
|  | RET | Communication port for the RET antenna, supporting RET signal transmission |
| (3) Indicator | RUN | For details, see RRU Indicators. |
|  | ALM |  |
|  | ACT |  |
|  | VSWR |  |
|  | CPRI0 |  |
|  | CPRI1 |  |

NOTE:

- The port for transmitting RET signals is determined by the software.

- Connect the CPRI0 port to the BBU by default in the single-mode scenario.

Parent Topic: RRU Introduction

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.2.5 RRU Indicators

This section describes six indicators on an RRU. They indicate the running status of the RRU.

For detailed positions of RRU indicators, see RRU Ports.

Table 1 describes RRU indicators.

| Table 1 RRU Indicators |  |  |  |
| --- | --- | --- | --- |
| Indicator | Color | Status | Meaning |
| RUN | Green | Steady on | The power input is available, but the board is faulty. |
|  |  | Steady off | No power input is available, or the board is faulty. |
|  |  | Blinking (on for 1s and off for 1s) | The board is running properly. |
|  |  | Blinking (on for 0.125s and off for 0.125s) | The board software is being loaded, or the board is not working. |
| ALM | Red | Steady on | Alarms are generated, and the module must be replaced. |
|  |  | Blinking (on for 1s and off for 1s) | Alarms are generated. The alarms may be caused by faults on the related board or ports. Therefore, you need to locate the fault before deciding whether to replace the module. |
|  |  | Steady off | No alarms are generated. |
| ACT | Green | Steady on | The board is working properly when TX channels are enabled or software is being loaded to a board that is not started. |
|  |  | Blinking (on for 1s and off for 1s) | The board is running with TX channels disabled. |
| VSWR | Red | Steady off | No voltage standing wave ratio (VSWR) alarm is generated. |
|  |  | Blinking (on for 1s and off for 1s) | VSWR alarms are generated on the B T/R port. |
|  |  | Steady on | VSWR alarms are generated on the A T/R port. |
|  |  | Blinking (on for 0.125s and off for 0.125s) | VSWR alarms are generated on the A T/R and B T/R ports. |
| CPRI0 | Red and green | Steady green | The CPRI link is running properly. |
|  |  | Steady red | An optical module fails to receive or transmit signals possibly because the optical module is faulty or the optical fiber is broken. |
|  |  | Blinking red (on for 1s and off for 1s) | The CPRI link is out of lock because of faults on the mutual lock of dual-mode clock sources or mismatched data rates on CPRI ports. |
|  |  | Steady off | The optical module cannot be detected or is powered off. |
| CPRI1 | Red and green | Steady green | The CPRI link is running properly. |
|  |  | Steady red | An optical module fails to receive or transmit signals possibly because the optical module is faulty or the optical fiber is broken. |
|  |  | Blinking red (on for 1s and off for 1s) | The CPRI link is out of lock because of faults on the mutual lock of dual-mode clock sources or mismatched data rates on CPRI ports. |
|  |  | Steady off | The optical module cannot be detected or is powered off. |

NOTE:

VSWR indicators on RRUs providing one or more than two TX channels have two types of status:

- Steady off: No VSWR alarm is generated.

- Steady on: VSWR alarms are generated on one or more ports.

Parent Topic: RRU Introduction

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.2.6 Optical Modules

An optical module transmits optical signals between an optical port and a fiber optic cable.

NOTE:

- The exteriors of an optical module and the label on an optical module in this section are for reference only. The actual exteriors may be different.

- Boards or RF modules supporting only the 1.25 Gbit/s CPRI port rate, for example, a GTMU or RRU3908 V1, cannot use 10 Gbit/s optical modules.

- A fiber optic cable must use the same type of optical modules on its two ends. If different types of optical modules are used by a fiber optic cable, performance risks may arise, for example, alarms, bit errors, and interrupted links.

- Only Huawei-certified optical modules meeting the following requirements can be used for Huawei wireless devices:

  - Requirements of devices on which optical modules are to be installed

  - Laser safety in the IEC 60825-1 standard

  - General safety in the IEC 60950-1 standard

- For SFP or QSFP optical modules certified and provided by Huawei, see Spare Parts Catalog.

The following figures show the exteriors of optical modules.

Figure 1 Exteriors of optical modules (SFP)

| a: Two-fiber bidirectional optical module | b: Single-fiber bidirectional optical module |
| --- | --- |

Figure 2 Exteriors of optical modules (QSFP)

| a: 100G SR4 optical module | b: Two-fiber bidirectional optical module | c: Single-fiber bidirectional optical module |
| --- | --- | --- |

| a: 100G BIDI optical module | b: Single-fiber bidirectional optical module |
| --- | --- |

NOTE:

QSFP optical modules are applicable only to BBUs, massive MIMO devices, and radio multiplex units (RMUs), but not to RRUs. In addition, only QSFP optical modules with a data rate of 40 Gbit/s are applicable to the RMUs.

- Label on an Optical Module

- Optical Module Type

#### Label on an Optical Module

There is a label on each optical module, which provides information such as the rate, wavelength, and transmission mode, as shown in the following figure.

Figure 3 Label on an optical module (SFP)

| (1) Rate | (2) Wavelength | (3) Transmission mode |
| --- | --- | --- |

Figure 4 Label on an optical module (QSFP)

| (1) Rate | (2) Wavelength | (3) Transmission mode |
| --- | --- | --- |

| (1) Rate | (2) Wavelength | (3) Transmission mode | (4) Connected module |
| --- | --- | --- | --- |

There are different types of optical modules which are configured depending on the actual application scenarios, see CPRI Fiber Optic Cable in 3900 Series Base Station Cables.

#### Optical Module Type

Optical modules can be divided into single- and multimode optical modules, which can be distinguished as follows:

- The puller of a single-mode optical module is blue and the puller of a multimode optical module is black or gray.

- The transmission mode is displayed as "SM" on the label of a single-mode optical module and "MM" on the label of a multimode optical module.

Parent Topic: BBU3900 and BBU3910 Boards

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.3 RRU Cables

This chapter describes RRU cables.

RRU Cable List

This section describes RRU cable connections.

RRU PGND Cable

An RRU PGND cable connects an RRU and a ground bar, ensuring the proper grounding of the RRU. The maximum length of an RRU PGND cable is 8 m (26.25 ft).

RRU Power Cable

The RRU power cable is a -48 V DC shielded cable. It feeds -48 V DC power to an RRU. The maximum length of power supply that an RRU power cable supports is 100 m (328.08 ft).

CPRI Fiber Optic Cable

CPRI fiber optic cables are classified into multimode fiber optic cables and single-mode fiber optic cables. They transfer CPRI signals.

RRU RF Jumper

The 1/2" RRU RF jumper transmits and receives RF signals between an RRU and an antenna. A fixed-length RF jumper used by an RRU is 2 m (6.56 ft), 3 m (9.84 ft), 4 m (13.12 ft), 6 m (19.68 ft), or 10 m (32.81 ft). A variable-length RF jumper used by an RRU has a maximum length of 10 m (32.81 ft).

RRU AISG Multi-Wire Cable

An RRU AISG multi-wire cable connects an RRU and an RCU to transmit control signals from a base station to an RET antenna. When the RRU is connected to the RET antenna, an AISG multi-wire cable transmits RS485 signals. The length of the AISG multi-wire cable is 5 m (16.40 ft).

RRU AISG Extension Cable

When the distance between an RRU and an RCU is longer than 5 m (16.4 ft), an AISG multi-wire cable is not long enough to connect the RRU and the RCU. In this case, an AISG extension cable is used to extend the AISG multi-wire cable for transmitting RS485 signals. The length of the AISG extension cable is 15 m (49.21 ft).

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.3.1 RRU Cable List

This section describes RRU cable connections.

Table 1 lists RRU cables.

| Table 1 RRU cables |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cable | One End |  | The Other End |  |
|  | Connector | Installation Position | Connector | Installation Position |
| RRU PGND Cable | OT terminal (M6, 16 mm2 or 0.025 in.2) | Ground terminal on the RRU | OT terminal (M8, 16 mm2 or 0.025 in.2) | Ground terminal on the ground bar |
| RRU Power Cable | Female fast connector (pressfit type) | NEG(-) and RTN(+) ports on the RRU | Depending on the power supply equipment | External power equipment |
| CPRI Fiber Optic Cable | DLC connector | CPRI0 port on the RRU | DLC connector | CPRI port on a board in the BBU or CPRI1 port on the upper-level RRU |
|  |  | CPRI1 port on the RRU |  | CPRI0 port on the lower-level RRU or CPRI port on a board in the BBU |
| RRU RF Jumper | 4.3-10 male connector | RF ports on the RRU | 4.3-10 male connector or DIN male connector NOTICE: Select cables based on the type of the connector at the other end of the cable on live networks. | Antenna system |
| RRU AISG Multi-Wire Cable | DB9 waterproof male connector | RET port on the RRU | Standard AISG female connector | Standard AISG male connector on the RCU or on the AISG extension cable |
| RRU AISG Extension Cable | Standard AISG male connector | Standard AISG female connector on the AISG multi-wire cable | Standard AISG female connector | Standard AISG male connector on the RCU |

Parent Topic: RRU Cables

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

Next topic >

# 6.3.5.1.3.2 RRU PGND Cable

An RRU PGND cable connects an RRU and a ground bar, ensuring the proper grounding of the RRU. The maximum length of an RRU PGND cable is 8 m (26.25 ft).

#### Exterior

A PGND cable is green or green and yellow with a cross-sectional area of 16 mm2 (0.025 in.2). An OT terminal is installed at each end of the cable. Figure 1 shows a PGND cable.

Figure 1 PGND cable

| (1) OT terminal (M6, 16 mm2 or 0.025 in.2) | (2) OT terminal (M8, 16 mm2 or 0.025 in.2) |
| --- | --- |

NOTE:

- If the customer prepares the PGND cable, a copper-core cable with a cross-sectional area of 16 mm2 (0.025 in.2) or larger is recommended.

- One OT terminal must be added to each end of the PGND cable onsite.

- You can determine the color of the cable and whether to use corresponding two-hole OT terminals based on local regulations.

Figure 2 shows a two-hole OT terminal.

Figure 2 Two-hole OT terminal

Parent Topic: RRU Cables

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.3.3 RRU Power Cable

The RRU power cable is a -48 V DC shielded cable. It feeds -48 V DC power to an RRU. The maximum length of power supply that an RRU power cable supports is 100 m (328.08 ft).

NOTE:

If a power device provided by the customer is used, the recommended specification of the circuit breaker on this power device is 20 A to 30 A.

#### Exterior

A female fast connector (pressfit type) needs to be added to one end of the RRU power cable and a corresponding terminal needs to be added to the other end based on the requirements of the connector on the external power device, as shown in Figure 1.

Figure 1 RRU power cable

| (1) -48 V DC power cable | (2) Shield layer | (3) Female fast connector (pressfit type) |
| --- | --- | --- |

The outer diameter of RRU power cables ranges from 9.9 mm to 14.4 mm, and Table 1 lists the specifications of an RRU power cable.

| Table 1 Specifications of an RRU power cable |  |  |  |  |
| --- | --- | --- | --- | --- |
| RRU Type | National Standard | Remote Distance | Cross-Sectional Area of the Cable | OCB Quantity |
| RRU5909s | European standard | 0 m to 100 m | 4 mm2 | 0 |
|  | American standard | 0 m to 100 m | 3.3 mm2 | 0 |

Table 2 lists the wire colors of an RRU power cable.

| Table 2 Wire Colors of an RRU power cable |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Cable | Wire | Wire Color in Most Regions |  | Wire Color in Other Regions |  |
|  |  | North American Standard | European Standard | UK | China |
| RRU power cable | RTN(+) | Black | Brown | Blue | Red |
|  | NEG(-) | Blue | Blue | Gray | Blue |

Parent Topic: RRU Cables

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.3.4 CPRI Fiber Optic Cable

CPRI fiber optic cables are classified into multimode fiber optic cables and single-mode fiber optic cables. They transfer CPRI signals.

A multimode fiber optic cable connects a BBU to an RRU or connect two RRUs. The maximum length of a multimode fiber optic cable connecting a BBU and an RRU is 150 m (492.12 ft). The default length of a multimode fiber optic cable connecting two RRUs is 10 m (32.81 ft). If the distance between two RRUs exceeds 10 m (32.81 ft), contact Huawei engineers.

The single-mode fiber optic cable can be divided into two types: single-mode fiber optic cable directly connecting a BBU to an RRU or connects two RRUs (shortened to directly connected single-mode fiber optic cable) and single-mode fiber optic cable connecting an ODF to a BBU or RRU (shortened to single-mode pigtail). The length of each single-mode fiber optic cable between the BBU and RRU or between RRUs cannot exceed the maximum allowed remote distance between the BBU and RRUs. The maximum length of the single-mode pigtail between an ODF and a BBU is 20 m (65.62 ft). The maximum length of the single-mode pigtail between an ODF and an RRU is 70 m (229.66 ft).

NOTE:

- The ODF is an outdoor transfer box for fiber optic cables, which interconnects the single-mode pigtail and trunk single-mode fiber optic cable.

- The ODF and trunk single-mode fiber optic cable are provided by the customer and must comply with the ITU G.652 standard.

- A multimode fiber optic cable is used together with a multimode optical module, and a single-mode pigtail is used together with a single-mode optical module.

#### Selection Principle

The following table describes the principles for selecting CPRI fiber optic cables.

| Table 1 Principles for selecting CPRI fiber optic cables |  |  |  |  |
| --- | --- | --- | --- | --- |
| Remote Distancea | CPRI Data Rate of the Optical Module (Gbit/s) | Selection Principle | Connects...to... | Remarks |
| ≤ 100 m (328.08 ft) | 2.5 or 4.9 | Multimode fiber optic cable | Connects a BBU to an RRU or connect two RRUs. | When multiple RRUs are cascaded, multimode fiber optic cables and single-mode fiber optic cables can be used together. |
|  | 9.8 | Directly connected single-mode fiber optic cable |  |  |
| 100 m (328.08 ft) < Remote distance ≤ 150 m (492.12 ft) | All | Recommended: single-mode fiber optic cable (single-mode pigtail and trunk single-mode fiber optic cable) | Connects a BBU to an RRU. | The single-mode pigtail at the RRU or BBU side is connected to the trunk single-mode fiber optic cable by using an ODF. |
|  | 2.5 or 4.9 | Multimode fiber optic cable | Connects a BBU to an RRU or connect two RRUs. | When multiple RRUs are cascaded, multimode fiber optic cables and single-mode fiber optic cables can be used together. |
|  | 9.8 | Directly connected single-mode fiber optic cable |  |  |
| > 150 m (492.12 ft) | - | Single-mode fiber optic cable (single-mode pigtail and trunk single-mode fiber optic cable) | Connects a BBU to an RRU. | The single-mode pigtail at the RRU or BBU side is connected to the trunk single-mode fiber optic cable by using an ODF. |
| ≤ 70 m (229.66 ft) | 40 or 100 | OM3 multimode fiber optic cable | Connects a BBU to an AAU. | Short-distance connection between the BBU and RRU |
| 70 m (229.66 ft) < Remote distance ≤ 100 m (328.08 ft) | 40 or 100 | OM4 multimode fiber optic cable |  |  |
| 100 m to 1000 m (328.08 ft to 3280.8 ft) | 40 or 100 | Directly connected single-mode fiber optic cable | Connects a BBU to an RRU. | - |
|  | 40 or 100 | Single-mode fiber optic cable (single-mode pigtail and trunk single-mode fiber optic cable) | Connects a BBU to an RRU. | The single-mode pigtail at the RRU or BBU side is connected to the trunk single-mode fiber optic cable by using an ODF. |

a: The remote distance between the BBU and RRUs refers to the total length of the cables between the BBU and the RRU that is farthest way from the BBU.

#### Exterior

Multimode fiber optic cable or directly connected single-mode fiber optic cable: The cable has a DLC connector at each end, as shown in the following figure.

Figure 1 Multimode fiber optic cable or directly connected single-mode fiber optic cable

| (1) DLC connector | (2) Breakout cable | (3) Label on the breakout cable |
| --- | --- | --- |

NOTE:

- When a fiber optic cable connects a BBU3900/BBU3910/BBU5900 to an RRU, the breakout cable on the BBU side is 0.34 m (1.12 ft) and the breakout cable on the RRU side is 0.03 m (0.098 ft).

- When a fiber optic cable connects a BBU3910A/BBU3910C to an RRU, the breakout cables on the BBU side and the RRU side are both 0.03 m (0.098 ft).

The following figure shows the connection of the multimode fiber optic cable or directly connected single-mode fiber optic cable between a BBU and an RRU as an example.

Figure 2 Connection of the multimode fiber optic cable or directly connected single-mode fiber optic cable between a BBU and an RRU

(1) Multimode fiber optic cable or directly connected single-mode fiber optic cable between a BBU and an RRU

Trunk fiber optic cable for an OFD06: The trunk fiber optic cable for an OFD06 connects a BBU to an OFD06 and has 12 LC connectors at each end, as shown in the following figure.

Figure 3 OFD06 trunk fiber optic cable

| (1) LC connector | (2) Breakout cable | (3) Label on the breakout cable |
| --- | --- | --- |

Single-mode pigtail: The single-mode pigtail has a DLC connector at one end and an FC, LC, or SC connector at the other end, as shown in the following figure.

Figure 4 Exterior of a single-mode pigtail

| (1) DLC connector | (2) Breakout cable | (3) Label on the breakout cable | (4) FC connector | (5) LC connector | (6) SC connector |
| --- | --- | --- | --- | --- | --- |

NOTE:

- When a single-mode pigtail connects a BBU to an ODF, the breakout cables on the BBU side and ODF side are 0.34 m (1.12 ft) and 0.8 m (2.62 ft), respectively.

- When a single-mode pigtail connects an RRU to an ODF, the breakout cables on the RRU side and ODF side are 0.03 m (0.098 ft) and 0.8 m (2.62 ft), respectively.

The following figure shows the connection of the single-mode pigtail.

Figure 5 Connection of the single-mode pigtail

| (1) Single-mode pigtail between a BBU and an ODF | (2) Single-mode pigtail between an RRU and an ODF |
| --- | --- |

The OM3 and OM4 multimode fiber optic cables have the same exterior, in which each end has an MPO connector. The MPO connector with a strap connects to the BBU and the MPO connector without a strap connects to the RRU, as shown in the following figure.

Figure 6 OM3/OM4 multimode fiber optic cable

| (1) MPO connector | (2) Strap | (3) MPO connector |
| --- | --- | --- |

#### Pin Assignment

The following table describes the labels on the breakout cables of a CPRI fiber optic cable and recommended cable connections.

| Table 2 Labels on the breakout cables of a CPRI fiber optic cable and recommended cable connections |  |  |  |
| --- | --- | --- | --- |
| Label on the Breakout Cable | Installation Position |  |  |
|  | Connection Between a BBU and an RRU or Between an OFD06 and an RRU | Connection Between RRUs | Connection Between a BBU or RRU and an ODF |
| 1A | CPRI RX port on the RRU | CPRI RX port on RRU 1 | RX port on the BBU or CPRI RX port on the RRU |
| 1B | CPRI TX port on the RRU | CPRI TX port on RRU 1 | TX port on the BBU or CPRI TX port on the RRU |
| 2A | OFD06 or TX port on the BBU | CPRI TX port on RRU 0 | ODF |
| 2B | OFD06 or RX port on the BBU | CPRI RX port on RRU 0 | ODF |

Parent Topic: CPRI Cables

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic

# 6.3.5.1.3.5 RRU RF Jumper

The 1/2" RRU RF jumper transmits and receives RF signals between an RRU and an antenna. A fixed-length RF jumper used by an RRU is 2 m (6.56 ft), 3 m (9.84 ft), 4 m (13.12 ft), 6 m (19.68 ft), or 10 m (32.81 ft). A variable-length RF jumper used by an RRU has a maximum length of 10 m (32.81 ft).

NOTE:

- When the distance between an RRU and an antenna is less than 10 m (32.81 ft), one end of the RF jumper is connected to an RF port at the bottom of the RRU, and the other end is connected to the antenna.

- When the distance between an RRU and an antenna is greater than 10 m (32.81 ft), it is recommended that one end of the RF jumper be connected to an RF port at the bottom of the RRU, the other end be connected to one end of the feeder, and the other end of the feeder be connected to the antenna.

- If the customer prepares the RF jumper, the length of the RF jumper should be as short as possible and not exceed 2 m (6.56 ft.).

#### Exterior

For an RF jumper, one end is connected to a 4.3-10 male connector, and the other end is connected to a connector based on onsite requirements, which is a 4.3-10 male connector or DIN male connector.

Figure 1 shows an RF jumper with two ends connected to 4.3-10 male connectors.

Figure 1 RF jumper

(1) 4.3-10 male connector

Parent Topic: RRU Cables

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.3.6 RRU AISG Multi-Wire Cable

An RRU AISG multi-wire cable connects an RRU and an RCU to transmit control signals from a base station to an RET antenna. When the RRU is connected to the RET antenna, an AISG multi-wire cable transmits RS485 signals. The length of the AISG multi-wire cable is 5 m (16.40 ft).

NOTE:

An RCU is a driving motor used for the phase shifter in the RET antenna. It receives control commands from a base station and runs the commands to drive the stepper motor. Using a gear, the stepper motor drives the adjustable phase shifter in the antenna and changes the downtilt angle.

#### Exterior

An AISG multi-wire cable has a waterproof DB9 male connector at one end and a standard AISG female connector at the other end, as shown in Figure 1.

Figure 1 AISG multi-wire cable

| (1) Waterproofed DB9 male connector | (2) Standard AISG female connector |
| --- | --- |

#### Pin Assignment

Table 1 describes the pin assignment for the wires of an AISG multi-wire cable.

| Table 1 Pin assignment for the wires of an AISG multi-wire cable |  |  |  |  |
| --- | --- | --- | --- | --- |
| X1 End (Pin of the Waterproofed DB9 Male Connector) | X2 End (Pin of the Standard AISG Female Connector) | Color | Type | Description |
| X1.1 | X2.1 | White and blue | Twisted pair | +12 V |
|  |  | Blue |  |  |
| X1.3 | X2.3 | White and orange | Twisted pair | RS485 B |
| X1.5 | X2.5 | Orange |  | RS485 A |
| X1.4 | X2.4 | White and green | - | GND |
| X1.9 and X1.4 are interconnected. | - | - | - | GND |
| - | X2.1 and X2.6 are interconnected. | - | - | +12 V |
| - | X2.4 and X2.7 are interconnected. | - | - | GND |

Parent Topic: RRU Cables

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.3.7 RRU AISG Extension Cable

When the distance between an RRU and an RCU is longer than 5 m (16.4 ft), an AISG multi-wire cable is not long enough to connect the RRU and the RCU. In this case, an AISG extension cable is used to extend the AISG multi-wire cable for transmitting RS485 signals. The length of the AISG extension cable is 15 m (49.21 ft).

#### Exterior

An AISG multi-wire cable has a standard AISG male connector at one end and a standard AISG female connector at the other end, as shown in Figure 1.

Figure 1 AISG extension cable

| (1) Standard AISG male connector | (2) Standard AISG female connector |
| --- | --- |

#### Pin Assignment

Table 1 describes the pin assignment for the wires of an AISG extension cable.

| Table 1 Pin assignment for the wires of an AISG extension cable |  |  |  |  |
| --- | --- | --- | --- | --- |
| X1 End (Pin of the Standard AISG Male Connector) | X2 End (Pin of the Standard AISG Female Connector) | Color | Type | Description |
| X1.1 | X2.1 | White and blue | Twisted pair | +12 V |
|  |  | Blue |  |  |
| X1.7 | X2.7 | White and orange | Twisted pair | DC Return |
|  |  | Orange |  |  |
| X1.3 | X2.3 | White and green | Twisted pair | RS485 B |
| X1.5 | X2.5 | Green |  | RS485 A |
| X1.6 | X2.6 | White and brown | Twisted pair | +24 V |
|  |  | Brown |  |  |

Parent Topic: RRU Cables

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic

# 6.3.5.1.4 RF Cable Connections for the RRU5909s

RF cable connections for the RRU vary depending on the configurations of the RRU and antenna.

#### Description of RF Cable Connections

This section describes the RF cable connections for the RRU serving a single sector. The following tables list the RF cable connections for the RRU.

| Table 1 RF cable connections for the RRU |  |  |  |
| --- | --- | --- | --- |
| RRU Model | Specifications of a Single RRU | Scenario | Illustration of Cable Connections |
| RRU5909s | For details, see the typical power configuration of the corresponding RRU in "Technical Specifications of RRUs" of Base Station Technical Description. | - 1T2R - 2T2R | See illustration 1 in Figure 1. |

#### Illustration of Cable Connections

Figure 1 RF cable connections for the RRU

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >

# 6.3.5.1.5 RRU Auxiliary Devices

This chapter describes RRU auxiliary devices.

#### Indoor Floor installation Support

An Indoor Floor installation Support (IFS06) is used for installing indoor RRUs.

For details about the exterior, specifications, and installation of an IFS06, see the Base Station Hardware Description and DBS3900 (ICR) Installation Guide.

#### Outdoor Pole Support

OFP06, short for Outdoor Flexible Pole For 6, is an outdoor pole support and provides a solution for extended installation of Huawei products on towers or building tops.

For details about the exterior and installation of an OFP06, see the OFP06 Installation Guide.

#### OCB

An Outdoor Cable Conversion Box (OCB) interconnects cables of different core diameters. Power cables shipped with RRUs cannot support long-distance power supply. Therefore, when power supply is far from the equipment, cables with large core diameters are used, and an OCB connects these cables and RRU power cables.

For details about the structure, functions, installation, and maintenance of an OCB, see the OCB User Guide or OCB–01M User Guide.

#### Power Module

Power module is used to convert external AC input power into -48 V DC for the DC RRU, including the following type:

- OPM15M(Ver.B): supported by V100R010C10 and later.

- OPM50M(Ver.B): supported by V100R010C10 and later.

For details about the structure, functions, installation, and maintenance of a Power Module, see the OPM15M(Ver.B) User Guide or OPM50M(Ver.B) User Guide.

#### Distribution Equipment

The ODM03D is small distribution equipment, which supports RRU capacity expansion, power cable reuse, and fiber optic cable reuse.

For details about the structure, functions, installation, and maintenance of an ODM03D, see the ODM03D User Guide.

#### RMU

A RMU3900A (Outdoor Distribution Module) is a radio multiplex unit, which converges and deconverges CPRI data and saves CPRI fiber optic cables between a BBU and RRUs.

RMU3900A: supported by V100R012C00 and later.

For details about the structure, functions, installation, and maintenance of a RMU3900A, see the RMU3900A User Guide.

#### MEB

The Monitoring Extension Box (MEB) can convert external RS-485 communication signals and dry node alarm signals into power return circuit signals, implementing power supply monitoring.

MEB: supported by V100R010C10 and later.

For details about the structure, functions, installation, and maintenance of a MEB, see the MEB06D User Guide.

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic