# Typical Power Configuration for RRU5909s

## Contents

6.1.1.6.10.3.1 Typical Power Configuration for RRU5909s

# 6.1.1.6.10.3.1 Typical Power Configuration for RRU5909s

An RRU5909s is a 2TX RF module that can work in UMTS, LTE FDD, GL, or UL mode.

- Table 1 and Table 2 provide the typical power configuration for the RRU5909s (LTE FDD, 1800 MHz).

- Table 3 and Table 4 provide the typical power configuration for the RRU5909s (UMTS, 2100 MHz).

- Table 5 and Table 6 provide the typical power configuration for the RRU5909s (LTE FDD, 2100 MHz).

- Table 7 and Table 8 provide the typical power configuration for the RRU5909s (GL MSR, 1800 MHz).

- Table 9 and Table 10 provide the typical power configuration for the RRU5909s (UL MSR, 2100 MHz).

NOTE:

- If the power sharing feature is activated, assume that UEs in a cell are randomly located.

- If the RF module is placed at an altitude of 3500 to 4500 meters, its power reduces by 1 dB. If the RF module is placed at an altitude of 4500 to 6000 meters, its power reduces by 2 dB.

- Station spacing, frequency multiplexing factor, power control algorithm, and traffic model all affect the gains of dynamic power sharing. In most cases, network plans are designed on the basis of power specifications of dynamic power sharing.

- Before activating the dynamic power sharing feature, enable the DTX and power control functions. In GBSS8.1, the dynamic power sharing feature is mutually exclusive with the GBFD-113201 Concentric Cell, GBFD-114501 Co-BCCH Cell, GBFD-118001 BCCH Dense Frequency Multiplexing, and GBFD-117501 Enhanced Measurement Report (EMR) features. In GBSS9.0 and later versions, the dynamic power sharing feature can be used together with these features. However, the dynamic power sharing feature currently cannot be used together with the GBFD-117002 IBCA (Interference Based Channel Allocation), GBFD-117001 Flex MAIO, GBFD-118701 RAN Sharing, and GBFD-114001 Extended Cell features in GBSS8.1, GBSS9.0, and later versions.

- When two LTE carriers are configured, it is recommended that the power spectrum density (PSD) of the two carriers be set to the same value. Power spectrum density = Carrier output power/Carrier bandwidth (1.4 MHz and 3 MHz bandwidths are considered as 5 MHz bandwidth in this formula.)

| Table 1 Output power for the RRU5909s (LTE FDD, 1800 MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mode |  | Total Number of LTE FDD Carriers |  |  | Output Power per LTE FDD Carrier (W) |  |  | Bandwidth of LTE FDD Carrier (MHz) |  |
| LTE FDD |  | 1 (MIMO) |  |  | 2x40 |  |  | 1.4, 3 |  |
|  |  | 1 (MIMO) |  |  | 2x60 |  |  | 5, 10, 15, 20 |  |
|  |  | 2 (MIMO) |  |  | 2x30 |  |  | 1.4, 3, 5, 10, 15, 20 |  |
|  |  | 2 (MIMO) |  |  | Carrier 1: 2x20 Carrier 2: 2x40 |  |  | 1.4, 3, 5, 10, 15, 20 |  |
| Table 2 Configuration of RRU5909s PAs (LTE FDD, 1800 MHz) |  |  |  |  |  |  |  |  |  |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  | PA2 |  |  |  |
|  | Number of LTE FDD Carriers |  |  | Output Power per LTE FDD Carrier (W) |  | Number of LTE FDD Carriers |  |  | Output Power per LTE FDD Carrier (W) |
| LTE FDD 1 (MIMO) | 1 |  |  | 40 |  | 1 |  |  | 40 |
| LTE FDD 1 (MIMO) | 1 |  |  | 60 |  | 1 |  |  | 60 |
| LTE FDD 2 (MIMO) | 2 |  |  | 30 |  | 2 |  |  | 30 |
| LTE FDD 2 (MIMO) | 2 |  |  | Carrier 1: 20 Carrier 1: 40 |  | 2 |  |  | Carrier 1: 20 Carrier 1: 40 |
| Table 3 Output power for the RRU5909s (UMTS, 2100 MHz) |  |  |  |  |  |  |  |  |  |
| Mode |  |  | Total Number of UMTS Carriers |  |  |  | Output Power per UMTS Carrier (W) |  |  |
| UMTS |  |  | 1 |  |  |  | 60 |  |  |
|  |  |  | 2 |  |  |  | 60 |  |  |
|  |  |  | 3 |  |  |  | 30 |  |  |
|  |  |  | 4 |  |  |  | 30 |  |  |
|  |  |  | 5 |  |  |  | 20 |  |  |
|  |  |  | 6 |  |  |  | 20 |  |  |
|  |  |  | 7 |  |  |  | 15 |  |  |
|  |  |  | 8 |  |  |  | 15 |  |  |
|  |  |  | 1 (MIMO) |  |  |  | 2x60 |  |  |
|  |  |  | 2 (MIMO) |  |  |  | 2x30 |  |  |
|  |  |  | 3 (MIMO) |  |  |  | 2x20 |  |  |
|  |  |  | 4 (MIMO) |  |  |  | 2x15 |  |  |
|  |  |  | 2 + 1 (MIMO) |  |  |  | Non-MIMO carrier: 30 MIMO carrier: 2x30 |  |  |
|  |  |  | 4 + 1 (MIMO) |  |  |  | Non-MIMO carrier: 20 MIMO carrier: 2x20 |  |  |
|  |  |  | 2 + 2 (MIMO) |  |  |  | Non-MIMO carrier: 20 MIMO carrier: 2x20 |  |  |
|  |  |  | 6 + 1 (MIMO) |  |  |  | Non-MIMO carrier: 15 MIMO carrier: 2x15 |  |  |
|  |  |  | 4 + 2 (MIMO) |  |  |  | Non-MIMO carrier: 15 MIMO carrier: 2x15 |  |  |
|  |  |  | 2 + 3 (MIMO) |  |  |  | Non-MIMO carrier: 15 MIMO carrier: 2x15 |  |  |

| Table 4 Configuration of RRU5909s PAs (UMTS, 2100 MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  | PA2 |  |  |
|  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |  | Number of UMTS Carriers |  | Output Power per UMTS Carrier (W) |
| UMTS 1 | 1 |  | 60 |  | 0 |  | 0 |
| UMTS 2 | 1 |  | 60 |  | 1 |  | 60 |
| UMTS 3 | 2 |  | 30 |  | 1 |  | 30 |
| UMTS 4 | 2 |  | 30 |  | 2 |  | 30 |
| UMTS 5 | 3 |  | 20 |  | 2 |  | 20 |
| UMTS 6 | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 7 | 4 |  | 15 |  | 3 |  | 15 |
| UMTS 8 | 4 |  | 15 |  | 4 |  | 15 |
| UMTS 1 (MIMO) | 1 |  | 60 |  | 1 |  | 60 |
| UMTS 2 (MIMO) | 2 |  | 30 |  | 2 |  | 30 |
| UMTS 3 (MIMO) | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 4 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| UMTS 2 + 1 (MIMO) | 2 |  | 30 |  | 2 |  | 30 |
| UMTS 4 + 1 (MIMO) | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 2 + 2 (MIMO) | 3 |  | 20 |  | 3 |  | 20 |
| UMTS 6 + 1 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| UMTS 4 + 2 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| UMTS 2 + 3 (MIMO) | 4 |  | 15 |  | 4 |  | 15 |
| Table 5 Output power for the RRU5909s (LTE FDD, 2100 MHz) |  |  |  |  |  |  |  |
| Mode |  | Total Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth of LTE FDD Carrier (MHz) |  |
| LTE FDD |  | 1 (MIMO) |  | 2x60 |  | 5, 10, 15, 20 |  |
|  |  | 2 (MIMO) |  | 2x30 |  | 5, 10, 15, 20 |  |

| Table 6 Configuration of RRU5909s PAs (LTE FDD, 2100 MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) |  | PA1 |  |  |  | PA2 |  |  |  |
|  |  | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |  | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |  |
| LTE FDD 1 (MIMO) |  | 1 |  | 60 |  | 1 |  | 60 |  |
| LTE FDD 2 (MIMO) |  | 2 |  | 30 |  | 2 |  | 30 |  |
| Table 7 Output power for the RRU5909s (GL MSR, 1800 MHz) |  |  |  |  |  |  |  |  |  |
| Mode | Total Number of GSM Carriers |  | Total Number of LTE FDD Carriers |  | Output Power per GSM Carrier (W) |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth of LTE FDD Carrier (MHz) |
| GSM + LTE FDD | 1 |  | 1 (MIMO) |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 1 |  | 1 (MIMO) |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 1 |  | 1 (MIMO) |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 2 |  | 1 (MIMO) |  | 20 |  | 2x40 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 2 |  | 1 (MIMO) |  | 30 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 2 |  | 1 (MIMO) |  | 40 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 3 |  | 1 (MIMO) |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 4 |  | 1 (MIMO) |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 4 |  | 1 (MIMO) |  | 15 |  | 2x30 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 5 |  | 1 (MIMO) |  | 7.5 |  | 2x30 |  | 10 |
|  | 5 |  | 1 (MIMO) |  | 12 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 6 |  | 1 (MIMO) |  | 7.5 |  | 2x30 |  | 10 |
|  | 6 |  | 1 (MIMO) |  | 12 |  | 2x20 |  | 1.4, 3, 5, 10, 15, 20 |
|  | 1 |  | 2 (MIMO) |  | 20 |  | 2x20 |  | 1.4, 3, 5, 10 |
|  | 1 |  | 2 (MIMO) |  | 20 |  | Carrier 1: 2x30 Carrier 2: 2x10 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 1 |  | 2 (MIMO) |  | 30 |  | Carrier 1: 2x20 Carrier 2: 2x10 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 2 |  | 2 (MIMO) |  | 20 |  | Carrier 1: 2x10 Carrier 2: 2x30 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 2 |  | 2 (MIMO) |  | 20 |  | 2x20 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 3 |  | 2 (MIMO) |  | 10 |  | 2x20 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 3 |  | 2 (MIMO) |  | 15 |  | Carrier 1: 2x10 Carrier 2: 2x20 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 4 |  | 2 (MIMO) |  | 20 |  | 2x10 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 4 |  | 2 (MIMO) |  | 10 |  | 2x20 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |
|  | 4 |  | 2 (MIMO) |  | 15 |  | Carrier 1: 2x10 Carrier 2: 2x20 |  | Carrier 1: 1.4, 3, 5, 10, 15, 20 Carrier 2: 1.4, 3, 5, 10 |

| Table 8 Configuration of RRU5909s PAs (GL MSR, 1800 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  |  |  |  | PA2 |  |  |  |  |
|  | Number of GSM Carriers |  | Output Power per GSM Carrier (W) |  | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |  | Number of GSM Carriers | Output Power per GSM Carrier (W) | Number of LTE FDD Carriers |  | Output Power per LTE FDD Carrier (W) |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 0 | 0 | 1 |  | 40 |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 0 | 0 | 1 |  | 30 |
| GSM 1 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 0 | 0 | 1 |  | 20 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 20 |  | 1 | 40 |  | 1 | 20 | 1 |  | 40 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 30 |  | 1 | 30 |  | 1 | 30 | 1 |  | 30 |
| GSM 2 + LTE FDD 1 (MIMO) | 1 |  | 40 |  | 1 | 20 |  | 1 | 40 | 1 |  | 20 |
| GSM 3 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 1 | 20 | 1 |  | 20 |
| GSM 4 + LTE FDD 1 (MIMO) | 2 |  | 20 |  | 1 | 20 |  | 2 | 20 | 1 |  | 20 |
| GSM 4 + LTE FDD 1 (MIMO) | 2 |  | 15 |  | 1 | 30 |  | 2 | 15 | 1 |  | 30 |
| GSM 5 + LTE FDD 1 (MIMO) | 3 |  | 7.5 |  | 1 | 30 |  | 2 | 7.5 | 1 |  | 30 |
| GSM 5 + LTE FDD 1 (MIMO) | 3 |  | 12 |  | 1 | 20 |  | 2 | 12 | 1 |  | 20 |
| GSM 6 + LTE FDD 1 (MIMO) | 3 |  | 7.5 |  | 1 | 30 |  | 3 | 7.5 | 1 |  | 30 |
| GSM 6 + LTE FDD 1 (MIMO) | 3 |  | 12 |  | 1 | 20 |  | 3 | 12 | 1 |  | 20 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | 20 |  | 0 | 0 | 2 |  | 20 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | Carrier 1: 30 Carrier 2: 10 |  | 0 | 0 | 2 |  | Carrier 1: 30 Carrier 2: 10 |
| GSM 1 + LTE FDD 2 (MIMO) | 1 |  | 30 |  | 2 | Carrier 1: 20 Carrier 2: 10 |  | 0 | 0 | 2 |  | Carrier 1: 20 Carrier 2: 10 |
| GSM 2 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | Carrier 1: 10 Carrier 2: 30 |  | 1 | 20 | 2 |  | Carrier 1: 10 Carrier 2: 30 |
| GSM 2 + LTE FDD 2 (MIMO) | 1 |  | 20 |  | 2 | 20 |  | 1 | 20 | 2 |  | 20 |
| GSM 3 + LTE FDD 2 (MIMO) | 2 |  | 10 |  | 2 | 20 |  | 1 | 10 | 2 |  | 20 |
| GSM 3 + LTE FDD 2 (MIMO) | 2 |  | 15 |  | 2 | Carrier 1: 10 Carrier 2: 20 |  | 1 | 15 | 2 |  | Carrier 1: 10 Carrier 2: 20 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 20 |  | 2 | 10 |  | 2 | 20 | 2 |  | 10 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 10 |  | 2 | 20 |  | 2 | 10 | 2 |  | 20 |
| GSM 4 + LTE FDD 2 (MIMO) | 2 |  | 15 |  | 2 | Carrier 1: 10 Carrier 2: 20 |  | 2 | 15 | 2 |  | Carrier 1: 10 Carrier 2: 20 |
| Table 9 Output power for the RRU5909s (UL MSR, 2100 MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| Mode |  | Total Number of UMTS Carriers |  | Total Number of LTE FDD Carriers |  |  | Output Power per UMTS Carrier (W) |  | Output Power per LTE FDD Carrier (W) |  | Bandwidth of LTE FDD Carrier (MHz) |  |
| UMTS + LTE FDD |  | 1 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 1 (MIMO) |  |  | 30 |  | 2x30 |  | 5, 10, 15, 20 |  |
|  |  | 3 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 1 (MIMO) |  |  | 20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 1 (MIMO) |  | 1 (MIMO) |  |  | 2x30 |  | 2x30 |  | 5, 10, 15, 20 |  |
|  |  | 2 (MIMO) |  | 1 (MIMO) |  |  | 2x20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 3 (MIMO) |  | 1 (MIMO) |  |  | 2x15 |  | 2x15 |  | 5, 10, 15, 20 |  |
|  |  | 6 |  | 1 (MIMO) |  |  | 15 |  | 2x15 |  | 5, 10, 15, 20 |  |
|  |  | 2 |  | 2 (MIMO) |  |  | 20 |  | 2x20 |  | 5, 10, 15, 20 |  |
|  |  | 4 |  | 2 (MIMO) |  |  | 15 |  | 2x15 |  | 5, 10, 15, 20 |  |

| Table 10 Configuration of RRU5909s PAs (UL MSR, 2100 MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration Scenario (Total Number of Carriers) | PA1 |  |  |  | PA2 |  |  |  |
|  | Number of UMTS Carriers | Output Power per UMTS Carrier (W) | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) | Number of UMTS Carriers | Output Power per UMTS Carrier (W) | Number of LTE FDD Carriers | Output Power per LTE FDD Carrier (W) |
| UMTS 1 + LTE FDD 1 (MIMO) | 1 | 30 | 1 | 30 | 0 | 0 | 1 | 30 |
| UMTS 2 + LTE FDD 1 (MIMO) | 1 | 30 | 1 | 30 | 1 | 30 | 1 | 30 |
| UMTS 3 + LTE FDD 1 (MIMO) | 2 | 20 | 1 | 20 | 1 | 20 | 1 | 20 |
| UMTS 4 + LTE FDD 1 (MIMO) | 2 | 20 | 1 | 20 | 2 | 20 | 1 | 20 |
| UMTS 1 (MIMO) + LTE FDD 1 (MIMO) | 1 | 30 | 1 | 30 | 1 | 30 | 1 | 30 |
| UMTS 2 (MIMO) + LTE FDD 1 (MIMO) | 2 | 20 | 1 | 20 | 2 | 20 | 1 | 20 |
| UMTS 3 (MIMO) + LTE FDD 1 (MIMO) | 3 | 15 | 1 | 15 | 3 | 15 | 1 | 15 |
| UMTS 6 + LTE FDD 1 (MIMO) | 3 | 15 | 1 | 15 | 3 | 15 | 1 | 15 |
| UMTS 2 + LTE FDD 2 (MIMO) | 1 | 20 | 2 | 20 | 1 | 20 | 2 | 20 |
| UMTS 4 + LTE FDD 2 (MIMO) | 2 | 15 | 2 | 15 | 2 | 15 | 2 | 15 |

Parent Topic: Typical Power Configurations for RRUs

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

Next topic >