# RRU3959a Technical Specifications

## Contents

6.1.1.6.6.19 RRU3959a Technical Specifications

# 6.1.1.6.6.19 RRU3959a Technical Specifications

An RRU3959a is a multimode remote radio unit (RRU). Adopting the software-defined radio (SDR) technology, an RRU3959a can work in different modes through software configurations.

#### Supported RATs and Frequency Bands

The following table lists the RATs and frequency bands supported by an RRU3959a.

| Table 1 RATs and frequency bands supported by an RRU3959a |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Frequency Band (MHz) | Receive Frequency Band (MHz) | Transmit Frequency Band (MHz) | RAT | IBW (MHz) |
| RRU3959a | 900 EGSM | 880 to 915 | 925 to 960 | GSM, UMTS, LTE FDD, LTE NB-IoT, GU, GL, UL, GM, UM, LM, and GUL | 35 |
|  | 900 PGSM | 890 to 915 | 935 to 960 | GSM, UMTS, LTE FDD, LTE NB-IoT, GU, GL, UL, GM, UM, LM, and GUL | 25 |
|  | 1800 | 1710 to 1785 | 1805 to 1880 | GSM, LTE FDD, LTE NB-IoT, GL, GM, LM, and GLM | 45 |
|  | 2100 | 1920 to 1980 | 2110 to 2170 | UMTS, LTE FDD, and UL | 60 |

#### RF Specifications

The following table lists radio frequency (RF) specifications of an RRU3959a.

NOTE:

- ATBR in the Transmit and Receive Channels column indicates that this RF module has A transmit channels and B receive channels.

- CxD W in the Output Power column indicates that this RF module is configured with C transmit channels and the maximum output power of each transmit channel is D W.

- EF MSR indicates that E and F data is carried on the same transmit channel of an RF module.

- EF non-MSR indicates that E and F data is carried on two different transmit channels of an RF module.

- GSM receiver sensitivity is measured, as recommended in 3GPP TS 51.021, in the central band (80% of the full band) at the antenna connector on condition that the channel rate reaches 13 kbit/s and the bit error rate (BER) is not higher than 2%.

- UMTS receiver sensitivity is measured, as recommended in 3GPP TS 25.104, over the full band at the antenna connector on condition that the channel rate reaches 12.2 kbit/s and the BER is not higher than 0.001.

- LTE FDD receiver sensitivity is measured, as recommended in 3GPP TS 36.104, under a 5 MHz channel bandwidth based on the FRC A1-3 in Annex A.1 (QPSK, R = 1/3, 25 RBs) standard.

- LTE (NB-IoT) receiver sensitivity is measured, as recommended in 3GPP TS 36.104, under a 200 KHz channel bandwidth and a 15 KHz subcarrier spacing based on the FRC A14-1 in Annex A.14 (π/2 BPSK, R = 1/3, 1 RB) standard.

| Table 2 RF specifications of an RRU3959a |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type | Transmit and Receive Channels | Capacity | Receiver Sensitivity (dBm) |  |  | Output Power | Power Consumption |
|  |  |  | Receiver Sensitivity with One Antenna | Receiver Sensitivity with Two Antennas | Receiver Sensitivity with Four Antennas |  |  |
| RRU3959a | 2T2R | GSM: 8 TRXs UMTS: - 6 carriers without MIMO - 4 carriers with MIMO LTE FDD: 2 carriers (The bandwidth per carrier is 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz, or 20 MHz.) LTE NB-IoT: 1 carrier | GSM: - 900 MHz EGSM: -113.8 - 900 MHz PGSM: -113.8 - 1800 MHz: -114.0 UMTS: - 900 MHz EGSM: -125.9 - 900 MHz PGSM: -125.9 - 2100 MHz: -126.1 LTE FDD: - 900 MHz EGSM: -106.9 - 900 MHz PGSM: -106.9 - 1800 MHz: -106.6 - 2100 MHz: -106.9 LTE NB-IoT: - 900 MHz EGSM: -127.3 - 900 MHz PGSM: -127.3 - 1800 MHz: -127.3 | GSM: - 900 MHz EGSM: -116.6 - 900 MHz PGSM: -116.6 - 1800 MHz: -116.8 UMTS: - 900 MHz EGSM: -128.7 - 900 MHz PGSM: -128.7 - 2100 MHz: -128.9 LTE FDD: - 900 MHz EGSM: -109.7 - 900 MHz PGSM: -109.7 - 1800 MHz: -109.4 - 2100 MHz: -109.7 LTE NB-IoT: - 900 MHz EGSM: -130.1 - 900 MHz PGSM: -130.1 - 1800 MHz: -130.1 | GSM: - 900 MHz EGSM: -119.3 (theoretical value) - 900 MHz PGSM: -119.3 (theoretical value) - 1800 MHz: -119.5 (theoretical value) UMTS: - 900 MHz EGSM: -131.4 - 900 MHz PGSM: -131.4 - 2100 MHz: -131.6 LTE FDD: - 900 MHz EGSM: -112.4 - 900 MHz PGSM: -112.4 - 1800 MHz: -112.1 - 2100 MHz: -112.4 LTE NB-IoT: - 900 MHz EGSM: -132.8 - 900 MHz PGSM: -132.8 - 1800 MHz: -132.8 | The RRU3959a supports the maximum power configuration of 2x60 W. For typical power configurations, see Typical Power Configuration for RRU3959a. | See the RRU3959a sheet in 3900 Series Base Station Power Consumption Specifications. |

#### Engineering Specifications

The following table lists the equipment specifications of an RRU3959a.

| Table 3 Equipment specifications of an RRU3959a |  |  |  |
| --- | --- | --- | --- |
| Type | Input Power | Dimensions (H x W x D) | Weight (kg) |
| RRU3959a | -48 V DC; voltage range: -36 V DC to -57 V DC NOTE: The RRU3959a supports AC power supply when connected to an external AC/DC power module or an OPM15M. For details, see AC/DC Power Module User Guide and OPM15M User Guide. | 400 mm x 300 mm x 100 mm (without the housing) | 14 (without the housing) |

The following table lists the environmental specifications of an RRU3959a.

| Table 4 Environmental specifications of an RRU3959a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Operating Temperature | Relative Humidity | Absolute Humidity | Atmospheric Pressure |
| RRU3959a | - With solar radiation: -40°C to +50°C - Without solar radiation: -40°C to +55°C NOTE: According to the installation scenario, traffic load, and carrier configuration, the output power of some RRUs may temporarily decrease when they work within the range of 10°C higher than the allowed highest temperature. | 5% RH to 100% RH | 1 g/m3 to 30 g/m3 | 70 kPa to 106 kPa |

The following table lists the compliance standards for an RRU3959a.

| Table 5 Compliance standards for an RRU3959a |  |
| --- | --- |
| Item | Standard |
| RF | - 3GPP TS 45.005 - 3GPP TS 25.141 - 3GPP TS 36.141 - 3GPP TS 37.141 |
| Operating environment | ETSI EN 300019-1-4 V2.1.2 (2003-04) Class 4.1: "Non-weatherprotected locations" |
| Storage environment | ETSI EN300019-1-1 V2.1.4 (2003-04) class1.2 "Weather protected, not temperature-controlled storage locations" NOTE: - The validity period is one year. - The product can function properly within the validity period if the storage environment meets the preceding standards. |
| Shockproof protection | NEBS GR63 zone4 |
| Ingress Protection Rating | IP65 |
| Surge protection | - IEC 62305-1 Protection against lightning - Part 1:General principles - IEC 62305-3 Protection against lightning - Part 3:Physical damage to structures and life hazard - IEC 62305-4 Protection against lightning - Part 4:Electrical and electronic systems within structures - ITU-T K.35 Bonding configurations and earthing at remote electronic sites - ITU-T K.56 Protection of radio base stations against lightning discharges - ITU-T K.97 Lightning protection of distributed base stations - ETSI EN 300 253 Environmental Engineering(EE):Earthing and bonding configuration inside telecommunications centers - YD/T 2324-2011 Lightning protection requirements and test methods for Radio Base Stations (RBSs) - GB 50689-2011 Code for design of lightning protection and earthing engineering for telecommunication bureaus (stations) |

Table 6 lists the surge protection specifications of ports on an RRU3959a.

NOTE:

- Unless otherwise specified, the surge protection specifications depend on the surge waveform of 8/20 μs.

- All the surge current items, unless otherwise specified as Maximum discharge current, refer to Nominal discharge current.

| Table 6 Surge protection specifications of ports on an RRU3959a |  |  |
| --- | --- | --- |
| Port | Surge Protection Mode | Surge Protection Specifications |
| Power supply port | Surge current | 20kA |

#### Specifications of CPRI Ports

The following table lists the specifications of CPRI ports on an RRU3959a.

| Table 7 Specifications of CPRI ports on an RRU3959a |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Number of CPRI Ports | CPRI Data Rate (Gbit/s) | Topology | Cascading Level | Maximum Distance from a BBU (km) |
| RRU3959a | 2 | 1.25, 2.5, 4.9, or 9.8 | Star, chain, or dual-star | CPRI MUX: - GU: 6 - GL: 4 - UL: 4 - GUL: 4 - GM: 4 - UM: 4 - LM: 4 - GLM: 4 | GU: 40 GL/UL/GUL: The maximum distance from the BBU varies with the types of LTE BBPs configured. - When the LBBPc, or LBBPd1 is configured: 20 - When the LBBPd2, UBBPd3, UBBPd4, UBBPd5, UBBPd6, or UBBPe is configured: 40 - When the LBBPd3 is configured - 40 if the cell number is less than or equal to 3 - 20 if the cell number is greater than or equal to 4 GM/UM/LM/GLM: 20 |

#### Antenna Capability

The following table lists the antenna capability of an RRU3959a.

| Table 8 Antenna capability of an RRU3959a |  |  |
| --- | --- | --- |
| Type | TMA Support | Supported RET Antenna |
| RRU3959a | Supported | AISG2.0 and AISG1.1 |

NOTE:

For RF modules supporting RET antennas, the feeding voltage is 12 V and feeding current is 2.3 A.

Parent Topic: Technical Specifications of RRUs

Huawei Proprietary and Confidential Copyright © Huawei Technologies Co., Ltd.

Huawei Proprietary and Confidential
Copyright © Huawei Technologies Co., Ltd.

< Previous topic Next topic >