Operator Field Test Display

6110/5110, 6130/5130, 6150, 8810

| Status: | VERSION : 23, 03-Jul-98 |
| --- | --- |
| Filename: | dct3opft.doc |

## CONTENTS

1 Introduction

1.1 Changes for 61xx series (compared to 8110i series)

2 Using the field test display

2.1 Activating the Field Test Display

2.2 Deactivating the Field Test Display

2.3 Field test modes

2.4 Signal and battery level

2.5 Reserving SIM SCM locations (recommendations)

3 Information on the displays

3.1 Signalling displays, which can be visible to network operators

3.1.1  Display 1:   Information of the serving cell

3.1.2  Display 2:   More information of the serving cell

3.1.3  Display 3:   Information of the serving cell,

1st and 2nd neighbour

3.1.4  Display 4:   Information of the 3rd, 4th and 5th neighbour

3.1.5  Display 5:   Information of the 6th, 7th and 8th neighbour

3.1.6  Display 6:   Network selection display

3.1.7  Display 7:   System information bits for serving cell.

3.1.8  Display 8:   HSCSD display 1

3.1.9  Display 9:   HSCSD display 2

3.1.10 Display 10:  Values of paging repeat period, TMSI, periodic

location update timer, AFC and AGC

3.1.11 Display 11:  Network parameters

3.1.12 Display 12:  Ciphering, hopping DTX status and IMSI

3.1.13 Display 13:  Uplink DTX switching display

3.1.14 Display 14:  Toggle Screening Indicator

3.1.15 Display 17:  Switch BTS_TEST status

3.1.16 Display 18:  Lights status control

3.1.17 Display 19:  Toggle Cell Barred Status

1   Introduction

This document specifies the features of 61xx series Field Test Display.

Field Test Display, i.e. engineering test mode, is a configurable software

feature which is available through MMI menu.

Field Test Display is useful for testing mobile phones during development

or for verifying the operation of the network.

1.1 Changes for 61xx series (compared to 8110I series)

General changes:

- 12 character lines (13 in HD843).

- MNC (Mobile Network Code) in displays 6 and 11 has now 3 digits.

- SIM SCM is used instead of B-memory SCM in displays 17, 52 and 53.

Moved displays

- Display 2 moved to display 3, 3 to 4, 4 to 5 and 5 to 6.

Moved items

- Places of C1 and C2 changed in display 1.

- Reason of last call release copied to display 2

Removed displays:

- Displays that were not implemented in 8110 (i.e. removed only

from specification)

- Display 6: Start latest cell information saving

- Display 7: Information of the latest and 2nd latest

cell/time slot

- Display 8: Information of the 3rd and 4th latest

cell/time slot

- Display 9: Information of the fifth latest cell/time slot

New items:

- Display 21:

- Sum of membership function sets beliefs

New displays:

- Display 2:  More information of the serving cell

- items moved from display 1:

- paging mode

- maximum number of Random Access retransmission

- roaming indicator

- BSIC value

- Reason of last call release

- RX quality (full)

- C2 parameters

- Hopping flag, MAIO and HSN

- Display 8:  HSCSD display 1

- Display 9:  HSCSD display 2

2   Using the Field Test Display

When the Field Test Display is active, the phone works in a similar way as

without it, except that the arrow buttons (^,v) scroll through the various

tests, whereas they would scroll through the memory locations while the

Field Test Display is inactive. The test numbers that are not valid are

not scrolled.

The Field Test Display appears as a so called soft indicator, which means

that it is visible only if there is nothing else to display. For example,

once the first digit of a phone number has been dialled, the Field Test

Display disappears and the digit appears. After the whole number has been

dialled and a call setup has started, the number disappears and the Field

Test Display reverts.

When the Field Test Display is active and mobile originated call setup is

going, 'CALLING XXXXX' text is not shown. However, if some call divert is

active, 'note diverts' text is viewed. When keyguard is active, keyguard

texts (or icons) are disabled and Field Test Display is visible. Pressing

a key views normal keyguard texts. When user activates the keyguard,

keyguard note is shown shortly.

During proper power off (use of power key or RE-CHARGE indication)

some valuable data (e.g. the number of current field test display) is

stored onto the EEPROM. Next time when the phone is powered on the lastly

selected field test display will be automatically re-activated.

2.1  Activating the Field Test Display

The Field Test Display is located at the end of the main menu loop. It is

activated as follows:

- press the Menu button

- scroll in the main menu loop to field test display item

(NETWORK MONITOR)

- press the Select button

- enter the index number of the test to be activated at the

TEST NUMBER prompt

- confirm with the Ok button

The field test data will then appear in a moment. The index number of the

test will appear in the top right corner of the display. If given test

number is not valid "No Test" -text will appear to the display.

A quicker way to activate the Field Test Display is to use the menu

shortcut. The field test display is the last item in the main menu loop.

The shortcut activation of the field test display is done in the following

way:

MENU <number of the last item> <number of the desired field test display>

If the last number of the main menu loop is e.g. 8, in order to activate

test display number 20, just press Menu, 8, 20 and Ok in a sequence.

See chapter 2.5 for recommendations.

2.2  Deactivating the Field Test Display

The Field Test Display is deactivated as follows:

- press the Menu button

- scroll in the main menu loop to field test display item

- press the Select button

- enter 0 at the TEST NUMBER prompt

- confirm with the Ok button

2.3  Field test modes

There are three Field Test Display modes:

- execute mode

- data display mode

- help mode

Different modes are marked in this specification as follows:

**************        ++++++++++++++        ##############

*            *        +            +        #            #

*   Execute  *        +Data display+        #    Help    #

*    Mode    *        +    Mode    +        #    Mode    #

*            *        +            +        #            #

**************        ++++++++++++++        ##############

The execute mode is entered from the menu as described in 2.1 (either

scroll and select or shortcut). If the test index entered pertains to a

test that resets a timer (test 80) for example, then the timer is reset

as soon as the Ok button has been pressed in the menu, and the data

display mode takes over. In other words, the execute mode is of the

one-shot type. To run another test in the execute mode, the Field Test

Display menu must be re-activated.

During the data display mode, the field test data (e.g. carrier, power

level, cell) is visible on the main display. During the help mode, one

screen of instructions is shown for each test to make it easier to identify

the test in question. A long press of asterisk (*) is used to toggle

between these two modes.

The arrow keys (^,v) offer an easy way to switch to another test without

using the menu. However, the data display mode remains, i.e. nothing will

be executed or set on although such tests would be passed. This is to

prevent the user from accidentally clearing any valuable data. (see 3.7.2

for details and 2.5 for recommendations).

The help mode is also a non-execute mode. Display numbers have been selected

in such way that no 5-terminated test number is an execute display.

2.4 Signal and battery levels

When Field Test Display is active, normal signal and battery level

bars are visible.

2.5 Reserving SIM SCM locations (recommendations)

When starting to use a SIM-card in a phone with field test displays, it is

recommended to put some default data into SIM SCM locations that are used

by field test displays. This prevents accidental storing of phone numbers

and names into such locations.

SIM SCM     Recommended    Recommended     Location used

location    number         text (name)     by display

--------------------------------------------------------

33             0         BTS TEST           17

Reserving SIM SCM locations is not necessary if the user is sure that

he/she will never select these displays using menu shortcut (which

executes the display).

In 61xx series (DCT3) phones it is not possible to select the SIM

location number where the number and the name will be stored.

So, reserving a SIM location must be done by some PC software or,

for example, by 8110i (DCT2) phone.

3   Information on the displays

Length of a field is shown in the display picture by amount of the letters.

If a value is not available, 'x' is shown for all the digits. If data to

be displayed is shorter than the field reserved to show that data, text

is aligned left and numbers right.

3.1 Signalling displays, which can be visible to network operators

Those signalling displays which can be activated by the network operators

are numbered from 1 to 19. Before field test displays are visible, mobile

has to be modified. With the normal production mobile field test

displays are not working.

3.1.1  Display 1:   Information of the serving cell

++++++++++++++

+abbb ccc ddd+

+ e ff g mmmm+

+ nnn     ppp+

+    oooo    +

++++++++++++++

a       H, if carrier numbers are scrolled when hopping is on.

Otherwise ' '.

bbb     When mobile is on TCH:

DCH carrier number in decimal.

When mobile is NOT on TCH:

CH means carrier number in decimal.

If hopping is on, used channels are scrolled when display is

updated.

ccc     rx level in dBm, minus sign not shown if <=-100

ddd     tx power level. If transmitter is on, symbol * is shown in

front of the power level value.

e       Time Slot, range is 0 - 7

ff      Timing advance, range is 0 - 63

g       rx quality (sub), range is 0 - 7

mmmm    Radio Link Timeout value. If value is negative, 0 is shown.

Maximum value is 64. When mobile is NOT on TCH then xx is shown.

nnn     value of the path loss criterium (C1). Range is -99 - 999.

oooo    type of current channel

THR0 : TCH HR subchannel 0

THR1 : TCH HR subchannel 1

TFR  : TCH FR

TEFR : TCH EFR

F144 : TCH FR data channel, speed 14.4 kbps

F96  : TCH FR data channel, speed 9.6 kbps

F72  : TCH FR data channel, speed 7.2 kbps

F48  : TCH FR data channel, speed 4.8 kbps

F24  : TCH FR data channel, speed 2.4 kbps

H480 : TCH HR data channel, speed 4.8 kbps, subch 0

H481 : TCH HR data channel, speed 4.8 kbps, subch 1

H240 : TCH HR data channel, speed 2.4 kbps, subch 0

H241 : TCH HR data channel, speed 2.4 kbps, subch 1

FA   : TCH FR signalling only (FACCH) channel

FAH0 : TCH HR signalling only (FACCH) channel, subch 0

FAH1 : TCH HR signalling only (FACCH) channel, subch 1

SDCC : SDCCH

AGCH : AGCH

CCCH : CCCH

CBCH : CCCH and cell broadcast receiving on

BCCH : BCCH

SEAR : SEARCH

NSPS : MS is in No Serv Power Save state

ppp         value of the cell reselection criterium (C2).

Range is -99 - 999. If phone is phase 1 then C1 value is

shown.

HELP display:

##############

#CH RxL TxPwr#

#TS TA RQ RLT#

# C1      C2 #

#    CHT     #

##############

3.1.2  Display 2:   More information of the serving cell

++++++++++++++

+ aa b c Bdd +

+ ee f       +

+ ggg hh iii +

+ H=j mm nn  +

++++++++++++++

aa      paging mode

NO : normal paging

EX : extended paging

RO : paging reorganization

SB : same as before

b       maximum number of Random Access retransmission

c       roaming indicator, values are R or empty.

Bdd     Letter B and BSIC value, range is 0 - 63.

ee      Reason of last call release

Cause from messages disconnect and release complete.

f       RX quality (full), range is 0 - 7

ggg     Cell reselect offset, range 0 - 126 dB.

0 - 63 * 2 dB. 'xxx' in active mode.

hh      Temporary offset, range 0 - 60 dB.

0 - 7 * 10 dB. 70 dB means infinite time.

'xx' in active mode.

iii     Penalty time, range 0 - 620 s.

0 - 31 * 20 s. 'xxx' in active mode.

j       Hopping channel

0    Single RF channel

1    RF hopping channel

mm      mobile allocation index offset, MAIO

Range: 00 to 63 / xx when H=0

nn      hopping sequence number, HSN

Range: 00 to 63 / xx when H=0

HELP display:

##############

#PM RAR Ro BC#

#RelR QLF    #

#CRO TO PenT #

#H MAIO HSN  #

##############

3.1.3  Display 3:   Information of the serving cell, 1st and 2nd neighbour

++++++++++++++

+aaabbbcccddd+

+aaabbbcccddd+

+aaabbbcccddd+

+    ef gh   +

++++++++++++++

1. row: serving cell information

2. row: 1. neighbour information

3. row: 2. neighbour information

4. row, ef: 1. neighbour information

4. row, gh: 2. neighbour information

aaa     carrier number in decimal

bbb     C1 value, range is -99 - 999, displayed only in idle mode.

Instead of C1 value, letter 'B' and BSIC value will be

displayed in active mode.

ccc     rx level in dBm, minus sign not shown if <=-100

ddd     C2 value, range is -99 - 999

e,g     F is shown if cell is in a forbidden location area

otherwise location is empty.

f,h     B is Barred, N is normal priority and L is low priority

otherwise location is empty.

HELP display:

##############

#SCH C1 rx C2#

#1CH C1 rx C2#

#2CH C1 rx C2#

#    1N 2N   #

##############

3.1.4  Display 4:   Information of the 3rd, 4th and 5th neighbour

++++++++++++++

+aaabbbcccddd+

+aaabbbcccddd+

+aaabbbcccddd+

+  ef gh ij  +

++++++++++++++

1. row: 3. neighbour information

2. row: 4. neighbour information

3. row: 5. neighbour information

4. row, ef: 3. neighbour information

4. row, gh: 4. neighbour information

4. row, ij: 5. neighbour information

aaa     carrier number in decimal

bbb     C1 value, range is -99 - 999, displayed only in idle mode.

Instead of C1 value, letter 'B' and BSIC value will be

displayed in active mode.

ccc     rx level in dBm, minus sign not shown if <=-100

ddd     C2 value, range is -99 - 999

e,g,i   F is shown if cell is in a forbidden location area

otherwise location is empty.

f,h,j   B is Barred, N is normal priority and L is low priority

otherwise location is empty.

HELP display:

##############

#3CH C1 rx C2#

#4CH C1 rx C2#

#5CH C1 rx C2#

#  3N 4N 5N  #

##############

3.1.5  Display 5:   Information of the 6th, 7th and 8th neighbour

++++++++++++++

+aaabbbcccddd+

+aaabbbcccddd+

+aaabbbcccddd+

+  ef gh ij  +

++++++++++++++

1. row: 6. neighbour information

2. row: 7. neighbour information

3. row: 8. neighbour information

4. row, ef: 6. neighbour information

4. row, gh: 7. neighbour information

4. row, ij: 8. neighbour information

aaa     carrier number in decimal

bbb     C1 value, range is -99 - 999, displayed only in idle mode.

Instead of C1 value, letter 'B' and BSIC value will be

displayed in active mode.

ccc     rx level in dBm, minus sign not shown if <=-100

ddd     C2 value, range is -99 - 999

e,g,i   F is shown if cell is in a forbidden location area

otherwise location is empty.

f,h,j   B is Barred, N is normal priority and L is low priority

otherwise location is empty.

HELP display:

##############

#6CH C1 rx C2#

#7CH C1 rx C2#

#8CH C1 rx C2#

#  6N 7N 8N  #

##############

3.1.6  Display 6:   Network selection display

This display shows the last registered network country code and

network code as well as the codes for four forbidden networks

and the first 3 preferred networks.

++++++++++++++

+aaabb  aaabb+

+aaabb  aaabb+

+aaabb  aaabb+

+aaabb  aaabb+

++++++++++++++

If three digit MNC is used (DCS1900), display looks different:

++++++++++++++

+aaabbbaaabbb+

+aaabbbaaabbb+

+aaabbbaaabbb+

+aaabbbaaabbb+

++++++++++++++

1. row: last registered network -  1st forbidden network

2. row: 1st preferred network   -  2nd forbidden network

3. row: 2nd preferred network   -  3rd forbidden network

4. row: 3rd preferred network   -  4th forbidden network

aaa     country code coded in BCD

bbb     network code coded in BCD, third digit can be 'F'

HELP display:

##############

#LReg   1_For#

#1_Pre  2_For#

#2_Pre  3_For#

#3_Pre  4_For#

##############

3.1.7  Display 7:  System information bits for serving cell.

++++++++++++++

+E A H C I BR+

+a b c d e fg+

+ECSC 2Ter MB+

+  h    i   j+

++++++++++++++

a       1 is shown if emergency calls are supported, else 0

b       1 is shown if attach-detach-procedure is allowed, else 0

c       1 is shown if half rate channels are supported, else 0

d       1 is shown if C2 values are broadcasted, else 0

e       1 is shown if system information 7 and 8 are broadcasted,

else 0

f       1 is shown if cell broadcast is supported, else 0

g       1 is shown if re-establishment is supported, else 0

The following items are used only in dualband phones:

h       In idle mode 1 is shown if Early Classmark (ECSC) sending is

supported, else 0. In dedicated mode (conversation) X is shown.

i       In idle mode 1 is shown if 2-Ter messages are supported,

else 0. In dedicated mode (conversation) X is shown.

j       MultiBand reporting decimal value (0,1,2,3) is shown if

supported. This is shown both in idle and dedicated mode.

The following is picked from Phase2+ ETSI GSM 05.08 version 5.4.0,

Section 8.4.3 ""Additional cell reporting requirements for multi

band MS"".

For a multi band MS the number of cells, for each frequency band

supported, which shall be included in the measurement report is

indicated by the parameter, MULTIBAND_REPORTING. The meaning of

different values of the parameter is specified as follows:

Value   Meaning

0 (00)  Normal reporting of the six strongest cells, with known and

allowed NCC part of BSIC, irrespective of the band used.

1 (01)  The MS shall report the strongest cell, with known and allowed

NCC part of BSIC, in each of the frequency bands in the BA

list, excluding the frequency band of the serving cell.

The remaining positions in the measurement report shall be used

for reporting of cells in the band of the serving cell.

If there are still remaining positions, these shall be used to

report the next strongest identified cells in the other bands

irrespective of the band used.

2 (10)   The MS shall report the two strongest cells, with known and

allowed NCC part of BSIC, in each of the frequency bands in the

BA list, excluding the frequency band of the serving cell.

The remaining positions in the measurement report shall be used

for reporting of cells in the band of the serving cell.

If there are still remaining positions, these shall be used to

report the next strongest identified cells in the other bands

irrespective of the band used.

3 (11)   The MS shall report the three strongest cells, with known and

allowed NCC part of BSIC, in each of the frequency bands in the

BA list, excluding the frequency band of the serving cell.

The remaining positions in the measurement report shall be used

for reporting of cells in the band of the serving cell.

If there are still remaining positions, these shall be used to

report the next strongest identified cells in the other bands

irrespective of the band used.

HELP display:

##############

#Serving Cell#

#System Info #

#Bits        #

#            #

##############

3.1.8  Display 8:  not implememnted !

HELP display:

##############

#            #

#            #

#            #

#            #

##############

3.1.9  Display 9: not implememnted !

HELP display:

##############

#            #

#            #

#            #

#            #

##############

3.1.10  Display 10:  Values of paging repeat period, TMSI, periodic location

update timer, AFC and AGC

++++++++++++++

+TMSIaaaaaaaa+

+T321:bbb/ccc+

+PRP:d  ee ff+

+ ggggg  hhh +

++++++++++++++

aaaaaaaa    TMSI value in hex format

bbb         Current value of T3212 counter (range is 000 - 'ccc', where

1 means 6 min time.  So, if this value is 2 less than 'ccc'

then next periodic location updating will be made within

2 * 6 min = 12 minutes.

ccc         Timeout value of T3212 counter (range is 000 - 240, where 1

means 6 min time between location updates and 240 means

240 * 6 min = 24 h between location updates. 000 means that

periodic location update is not in use.)  This value is

received from the network.

d           Value of paging repeat period (range is 2 - 9, when paging

is in every second multiframe, mobile takes more current

than if it were in every 9th multiframe)

ee          Downlink signalling failure value. If value is negative,

0 is shown. Maximum value is 45. When mobile is on TCH

then xx is shown.

ff          Gain value on TCH/SDCCH, range is 0 - 93

ggggg       VCTCXO AFC DAC control, range is -1024 - 1023

hhh         Serving cell channel number

HELP display:

##############

#TMSI(hex)   #

#T3212ctr/tim#

#PaRP DSF AGC#

#  AFC   Ch  #

##############

3.1.11  Display 11:  Network parameters

++++++++++++++

+CC:aaa NCbbb+

+ LAC:ccccc  +

+ CH : dddd  +

+ CID:eeeee  +

++++++++++++++

aaa     MCC value in decimal (MCC=Mobile Country Code)

bbb     MNC value in decimal (MNC=Mobile Network Code)

Three digits are shown only in DCS1900.

In other systems only two digits are shown.

ccccc   LAC value in decimal (LAC=Location Area Code)

dddd    Serving cell channel number

eeeee   Cell Identifier in decimal format

LAC and CID may be in hex format in some projects.

Or even both decimal and hexadecimal formats may be

shown on the same line.

HELP display:

##############

# MCC    MNC #

#LocAreaCode #

#ServChannel #

#   CellId   #

##############

3.1.12  Display 12:  Ciphering, hopping, DTX status and IMSI

++++++++++++++

+CIPHER :aaa +

+HOPPING:bbb +

+DTX    :ccc +

+IMSI   :ddd +

++++++++++++++

aaa     ciphering value, OFF/A51/A52

bbb     hopping value, ON/OFF

ccc     DTX value ON/OFF

ddd     IMSI attach

ON  : IMSI attach on

OFF : IMSI attach off

These values are updated only on the TCH.

HELP display:

##############

#CipherValue #

#HoppingValue#

#DTXValue    #

#IMSIAttach  #

##############

3.1.13  Display 13:  Uplink DTX switching display

With this display it is possible to change MS to use DTX or not, if BS

allows MS to decide it.

This display must be activated from MENU to change DTX state. When MENU is

not active and the user is scrolling field test displays with NEXT and

PREVIOUS, the DTX state will not change.

**************

*aaaaaaaaaa  *

*DTX(DEF):bbb*

*DTX(BS) :ccc*

*            *

**************

aaaaaaaaaa  status of switched mode.

DTX:ON    : MS uses DTX

DTX:OFF   : MS does not use DTX

DTX:DEF   : MS use default state of DTX.

Defined in MS_PAR.H

NOTALLOWED: BS does not allow MS to decide if it uses

DTX or not.

bbb         default state of DTX. Defined in MS_PAR.H

The value is either ON or OFF

ccc         is DTX value from BS

MAY : BS allows MS to decide if it uses dtx or not

on uplink.

USE : BS controls MS to use dtx (on uplink)

NOT : BS controls MS not to use dtx (on uplink)

HELP display:

##############

#DTXMode     #

#DefaulDTXSta#

#DTXValFromBS#

#            #

##############

3.1.14  Display 14:  Toggle Screening Indicator

When selected, changes the value of Screening Indicator from 0 to 1 and

vice versa.

**************                   **************

*  SCREENING *                   *  SCREENING *

*  INDICATOR *                   *  INDICATOR *

*    IS 00   *                   *    IS 01   *

*            *                   *            *

**************                   **************

HELP display:

##############

#Use menu to #

#  change    #

# Screening  #

# indicator  #

##############

3.1.15  Display 17:  Switch BTS_TEST status

**************                   **************

*            *                   *            *

*  BTS TEST  *                   *  BTS TEST  *

*     ON     *                   *    OFF     *

*            *                   *            *

**************                   **************

Mobile is                         Mobile is

searching only                    behaving normally.

one frequency.                    Neighbour measurements

Neighbour measurements            are done.

are not done.

This display is used to toggle BTS_TEST status on EEPROM. If BTS_TEST

status is set on EEPROM each time the mobile sends a search list it uses

only the carrier number stored on SIM SCM-location 33. Also the neighbour

information from system information messages is ignored. If the BTS_TEST

status is not set, then the value of SIM SCM-location 33 is ignored and the

mobile behaves normally (i.e. does the neighbour measurements according the

GSM specifications).

To activate BTS tests perform following steps:

- Save desired channel number in SIM SCM-location 33.

- Select display 17 in execute mode

- Switch power off and on

If activation succeeded, there is text "BTS TEST ON" in display 17.

To deactivate BTS tests either select display 17 in execute mode or

save number 0 in SIM SCM-location 33 and switch power off and on.

NOTE! The display does not show the value of BTS_TEST status in EEPROM.

Although the value is set, bts test can be off. If there is not legal

carrier number in SIM location 33 (GSM: 1-124, DCS1800: 512-885) the

display shows that bts test is off. Also if the mobile was already

registered to some carrier before switching BTS_TEST status, the display

can show different value from the one in EEPROM.

See chapter 2.5 for recommendations.

HELP display:

##############

#Use menu to #

#toggle BTS  #

#test ON/OFF #

#            #

##############

3.1.16  Display 18:  Lights status control

If selected, lights are always on, otherwise light status is controlled by

UI.

**************                   **************

*            *                   *            *

*   LIGHTS   *                   *   LIGHTS   *

*    ON      *                   *    OFF     *

*            *                   *            *

**************                   **************

HELP display:

##############

#Use menu to #

#  toggle    #

#  lights    #

#  ON/OFF    #

##############

3.1.17 Display 19:  Toggle Cell Barred Status

**************         **************         **************

*            *         *            *         *            *

* CELL BARR  *         * CELL BARR  *         * CELL BARR  *

* ACCEPTED   *         * REVERSE    *         * DISCARD    *

*            *         *            *         *            *

**************         **************         **************

This test is meant to be used when some cells are tested prior taking them

into commercial use. By setting the barring on in the base station normal

GSM phones will not try to register these barred cells. By selecting cell

barring reversed, the MS will only use the cells to be tested. However, if

at the same time it is wanted that MS will be capable to use normal network

cell barring ignored can be set. Display 19 will show the cell barring

mode.

NOTE! If a cell has been selected before barring state is changed the

selected cell may have different barring state than what the display shows.

After reselection the cell barring state is working for sure.

HELP display:

##############

#Use menu to #

#toggle cell #

#barr status #

#DIS/ACC/REV #

##############